| Out of management Company confidential S800For BSP PS-CPU Detailed Control Specifications |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Revision symbol | Date | Item | Approver | Reviewer | Author | Issuing department | Registration code |
| A | 2017/3/22 | First edition |  |  | Hirota | Information Equipment Business Development Headquarters Electronic Technology Unit Development Center Module Development Department | S800-BSP_DD-27(01) |
| B | 2017/6/23 | Edition B |  |  | Hirota Miwa | Information Equipment Development Headquarters System Platform Development Center 2nd Platform Development Department | S800-BSP_DD-27(02) |
| C | 2017/09/13 | Edition C |  |  | Watanabe | Information Equipment Development Headquarters System Platform Development Center 2nd Platform Development Department | S800-BSP_DD-27(03) |

Revision history

| Revision symbol | Date | Revision details | Reason for revision | Person in charge |
| --- | --- | --- | --- | --- |
| A | 2016/07/22 | First edition | S800For-target supportNewly created | Hirota |
| B | 2016/12/20 | ■ CT issue response Addition of missing information when creating CT specifications Change to I2C register specifications Changes due to insufficient ROM capacity in Main section RTC malfunction response Bug fix | 5.1.4.1. Communication specifications 5.1.4.2. STM32CubeMX settings 5.1.6. RTC control 5.1.6.1. STM32CubeMX settings 5.1.5.3.3 I/O expansion 5.1.5.3. Flash ROM partitioning 5.3. Function list 5.4. Constant specifications 5.6.Structure specifications 5.9. Function specifications | Miwa |
| B | 2017/06/23 | ■Changes corresponding to hardware specification document revisions Change of reference specification document name Corrections due to hardware changes DescriptionAdditional information | 1.4. Related documents All chapters related to signal control 5.2.Outline sequence diagram 5.2.1. Overall sequence diagram 5.2.2. Power ON sequence diagram 5.2.3. Power OFF sequence diagram | Hirota |
| C | 2017/09/13 | ■Support for adding PS-CPU ・PowerOInstantaneous interruption handling at N time ・PSupport for PS-CPU program rewriting ・Checksum / version information support | 4.2 Source/folder list 5.1.5.3.3 I/O expansion Checksum / version 5.1.5.5 Program rewrite reset control 5.1.7 Instantaneous interruption handling at initial power ON 5.1.8 Checksum / version information 5.3. Function list 5.4. Constant list 5.7. Global variables 5.9.1.10 power_monitor_proc 5.9.1.13 s800_power_off 5.9.1.16 ap_power_en_proc 5.9.1.28 checksum_calc 5.9.2.1. main 5.9.2.9. msw_on_proc_iap 5.9.2.10 km_exti_callback_iap 5.9.2.11 km_it_init_iap | Watanabe |

《Table of Contents》

1. Introduction	7

1.1. Purpose	7

1.2. Scope of application	7

1.3. Overview	7

1.4. Related documents	7

2. Definition of terms	8

3. Hardware specifications	9

3.1. PS-CPUBasic information	9

3.1.1. Overview	9

3.1.2. Memory Map	9

3.2. Hardware Configuration Diagram	10

3.2.1. S800Platform Diagram	10

3.2.2. Connection Circuit	11

3.2.3. Power Circuit	12

3.2.1. Pin Assignment	13

3.3. Register Specifications	14

4. Software Configuration	15

4.1. Software Configuration Diagram	15

4.2. Source/Folder List	15

4.3. Development Environment	17

4.3.1. STM32CubeMX	17

4.3.1.1. Peripheral,PINConfiguration Method	17

4.3.1.2. Code Generation	18

4.3.2. μVision	19

4.3.2.1. Multi-Project	19

4.3.2.2. Build	19

4.3.2.3. Debugger	20

4.3.2.4. File Addition	20

5. Control Specifications	21

5.1. SWControl Overview	21

5.1.1. KMInput Signals	21

5.1.1.1. MONI_24V11	21

5.1.1.2. SB_PG	21

5.1.1.3. MC3.3VON_MONI	21

5.1.2. KMOutput Signals	22

5.1.2.1. IR_P_ON	22

5.1.2.2. ERP_SENSOR_ON	22

5.1.2.3. /RESET	22

5.1.2.4. SB_PWR_EN	22

5.1.2.5. MC_P_ON	22

5.1.2.6. /DISCHG	22

5.1.2.7. /RST_SLP2	22

5.1.2.8. /USB2_OE	23

5.1.2.9. MC_PWR_EN	23

5.1.2.10. WAKEUP	23

5.1.3. KMInterrupt Signals	23

5.1.3.1. /HRESET_REQ	23

5.1.3.2. AP_PWR_EN	23

5.1.3.3. POWER_MONITOR	23

5.1.3.4. MSW_ON	23

5.1.4. I2CControl	24

5.1.4.1. Communication Specifications	24

5.1.4.2. STM32CubeMXSettings	24

5.1.4.3. Control Sequence	26

5.1.4.4. PS-CPU 2ndAddress Specifications	27

5.1.5. IAPControl	33

5.1.5.1. IAPControl Base	33

5.1.5.2. Project Segmentation	33

5.1.5.3. FlashROMSegmentation	34

5.1.5.4. I2CCommunication	34

5.1.5.5. Program Rewrite Reset Control	35

5.1.6. RTCControl	36

5.1.6.1. STM32CubeMXSettings	36

5.1.7. Initial PowerONMomentary Power Interruption Handling at Startup	37

5.1.7.1. Initial PowerONSequence	37

5.1.8. Checksum / Version Information	38

5.2. Overview Sequence Diagram	39

5.2.1. Overall Sequence Diagram	39

5.2.2. PowerONSequence Diagram	40

5.2.3. PowerOFFSequence Diagram	41

5.3. Function list	42

5.3.1. mainProject	42

5.3.2. iapProject	44

5.4. Constant specifications	44

5.4.1. mainProject	44

5.4.2. iapProject	47

5.5. Enumeration Specifications	49

5.5.1. mainProject	49

5.5.2. iapProject	50

5.6. Structure Specifications	50

5.6.1. mainProject	50

5.6.1. iapProject	50

5.7. Global Variables	50

5.7.1. mainProject	51

5.7.2. iapProject	51

5.8. constDefinition	51

5.8.1. mainProject	51

5.9. Function specifications	53

5.9.1. mainProject	53

5.9.1.1. main	53

5.9.1.2. jump2iap	53

5.9.1.3. DeInit	54

5.9.1.4. change_cmd2array	54

5.9.1.5. io_extend_read	55

5.9.1.6. io_extend_write	55

5.9.1.7. io_extend_memory_write	56

5.9.1.8. internal_sts_init	56

5.9.1.9. mc3_3von_moni_proc	56

5.9.1.10. power_monitor_proc	57

5.9.1.11. hreset_req_proc	58

5.9.1.12. extend_output_init	58

5.9.1.13. s800_power_off	58

5.9.1.14. moni_24v_proc	59

5.9.1.15. s800_power_on	59

5.9.1.16. ap_power_en_proc	60

5.9.1.17. erp_sensor_proc	60

5.9.1.18. rst_usb_proc	60

5.9.1.19. mc_p_on_proc	61

5.9.1.20. ir_p_on_proc	62

5.9.1.21. mc_oe_proc	62

5.9.1.22. mc_pwr_en_proc	62

5.9.1.23. msw_on_proc	63

5.9.1.24. rx_comp	63

5.9.1.25. change_state	64

5.9.1.26. i2c_recv_first	65

5.9.1.27. i2c_recv_wait	66

5.9.1.28. checksum_calc	67

5.9.1.29. hreset_req_proc_handler	67

5.9.1.30. km_exti_callback	67

5.9.1.31. km_it_init	68

5.9.1.32. HAL_TIM_PeriodElapsedCallback	68

5.9.1.33. km_timer_set	69

5.9.1.34. km_timer_get_state	69

5.9.1.35. adc_moni_5v_start_proc	70

5.9.1.36. adc_moni_5v_off_proc	70

5.9.1.37. BSP_PWR_enter_stopmode	70

5.9.1.38. BSP_PWR_enter_sleepmode	71

5.9.2. IAPProject	71

5.9.2.1. main	71

5.9.2.2. rx_comp	72

5.9.2.3. change_state	72

5.9.2.4. i2c_recv_first	72

5.9.2.5. i2c_recv_wait	73

5.9.2.6. flash_write_32	73

5.9.2.7. main_project_flash_erase	74

5.9.2.8. check_sum_calc	74

5.9.2.9. msw_on_proc_iap	75

5.9.2.10. km_exti_callback_iap	75

5.9.2.11. km_it_init_iap	76

6. Verification	77

6.1. Execution environment	77

6.2. Verification viewpoints	77

6.2.1. UTVerification execution	77

6.2.2. Startup	77

6.2.3. Various interrupt checks.	77

6.2.4. I2CCommunication	77

6.2.5. Port control	77

6.2.6. IAP	77

6.2.7. Momentary power interruption handling	77

# Introduction

## Purpose

This specification documentS800ForBSP PS-CPUshows the detailed control for.

## Scope of application

This specification documentS800for BSPis applied to the development stage.

## Overview

In this document,PS-CPUthe hardware configuration of , andPS-CPUdescribes the detailed control of the following functions of .

- Environment setup, build procedure

- IAP control

- Extended IO control

- Control of various peripherals

It corresponds to the power SubCPU of conventional models (S72/73), but due to the change of platform, the functions have also been greatly changed.

Due to functional changes from conventional models (S72/73), it is used as an extension of some power control and I/O of the S800 platform, so the name is changed from power SubCPU to PS-CPU.

The major differences from conventional models(S72/73)are as follows

- Fine-grained power control is not performed

Power control of various devices is basically performed by LPPP, and since PS-CPU mainly controls as an extension of I/O.

- Momentary power interruption control is not performed

Reset control is performed according to instructions from the S800 side, but PS-CPU does not perform reset control by its own judgment.。

- Data is not saved in non-volatile areas

Power-saving control is basically performed by LPPP, PS-CPU side has little control, so there is no need to save the sequence process in non-volatile areas。

- A debug terminal is not implemented

Support may be added depending on man-hoursbut since UART itself is only used for debugging, it will be postponed.

## Related documents

| No. | File name | Contents |
| --- | --- | --- |
| 1 | \\10.128.16.110\data1\29P.System Integrated SoC Development\03_BSP\11.Reference Materials\04.PS-CPU STM32F0x1_2_8RM(RM0091)_Japanese_rev1.pdf | Datasheet for STM32Fx1 (Japanese version) |
| 2 | \\10.128.16.110\data1\29P.System Integrated SoC Development\02_Board\24_Power SubCPU control specification PS-CPU control specification(X).docx | PS-CPU Control Specification Created by HW Team |
| 3 | \\10.128.19.213\guest\29P.System Integration SoC Development\PS-CPU\Doc\Function Specifications Function Specifications.docx | Function specifications for drivers created by Mikawa BSP |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |

# Terminology Definitions

Definitions of special terms used in this specification are shown in the table below.

| Term | Contents |
| --- | --- |
| STM32F031C6T6 | STmicromicrocontroller. Low-power, high-performance microcontroller. |
| PS-CPU | CPU mounted in the S800 that controls part of the power supply and the extended I/O. The RTC built into this CPU is used with the same control as the conventional RTC. Abbreviation of Power Sub CPU, refers to the Cortex-M0 CPU Architecture: v6-M, CPU bit width: 32bit, number of cores: 1 |
| AP | Abbreviation of Application Processor Block where AP CM3/AP CA72 are located |
| AON | Abbreviation of Always On Power Island Block where LPPP/PMU are located, and it is always powered on in any state other than "Power OFF" |
| PMU | Abbreviation of Power Management Unit, refers to the Cortex-M3 CPU located in the AON block Architecture: v7-M, CPU bit width: 32bit, number of cores: 1 Performs power control according to instructions from LPPP. Different from PS-CPU. |
| IAP | in-application programmingabbreviation. IAP is a function that reprograms the microcontroller’s flash memory while the user program is running |
| CA72(CA72_0-3) | Located in the AP blockCortex-A72 CPU Architecture: v8-A, CPU bit width: 64bit, number of cores: 4 (QuadCore) |
| LPPP | Abbreviation of Low Power Packet Processor, located in the AON blockCortex-R4 CPU Architecture: v7-R, CPU bit width: 32bit, number of cores: 1 |
| STM32CubeMX | STM32CubeMX is a source code generation tool provided by STmicro。 A tool that allows easy PIN configuration and enabling/disabling of peripherals using a GUI. |
| μVision | μVision is an integrated development environment provided by ARM (Keil). Supports compilers, source management, ulinketc.and debuggers that use them. |

# Hardware Specifications

## PS-CPU Basic Information

### Overview

An overview of the PS-CPU is as follows.

| Model number | STM32F031C6T6 |
| --- | --- |
| Core | Cortex-M0 |
| CPU CLOCK | 48MHz |
| Number of pins | 48 pins |
| RAM | 4KB |
| FLASH ROM | 32KB |
| SPI | 1 port |
| I2C | 1 port |
| ADC | 13 channels |

### Memory map

For the PS-CPU memory map, refer to the datasheet in the reference documents.

Refer to the following sections

2.2 Memory Configuration

2.2.1 Overview

2.2.2 Memory Map and Register Boundary Addresses

[References]

Reference 1: STM32F0x1_2_8RM(RM0091)_Japanese_rev1.pdf

## Hardware Configuration Diagram

### S800 Platform Diagram

The hardware block diagram of the S800 platform is as follows. The PS-CPU is in thered circlesection.

PS-The CPU mainly controls the power supply to peripheral components that work in conjunction with the S800 SB (South Bridge) section and AON (Always ON) section.

Detailed power control of the AP section and AON section is handled by the PMU, and not by the PS-CPU.

The main CPU (CA72), LPPP (R4) and PS-CPU communicateI2Cvia .Due to the nature of I2C communication, the main CPU and LPPP are the master devices, and the PS-CPU is the slave device.

[References]

Related document 2:PS-CPU control specification(X).docx

### Connection circuit

PS-CPUHardware configuration of the(connection circuit)is described.

The basic configuration is as shown in the following diagram.

[References]

Related document 2:PS-CPU control specification(X).docx

### Power circuit

The power circuit diagram is as follows.

[References]

Related document 2:PS-CPU control specification(X).docx

### Pin assignment

The pin assignment for S800 is as follows.

| Pin number | Pin type | Function | Signal name | Remarks |
| --- | --- | --- | --- | --- |
| 2 | PC13 | GPIO_Output | WAKEUP | 5.2.2.9.WAKEUP |
| 3 | PC14-OSC32_IN | RCC_OSC32_IN |  |  |
| 4 | PC15-OSC32_OUT | RCC_OSC32_OUT |  |  |
| 5 | PF0-OSC_IN | RCC_OSC_IN |  |  |
| 6 | PF1-OSC_OUT | RCC_OSC_OUT |  |  |
| 11 | PA1 | GPIO_EXTI1 | /HRESET_REQ | 5.2.4.1./HRESET_REQ |
| 12 | PA2 | GPIO_EXTI2 | AP_PWR_EN | 5.2.4.2.AP_PWR_EN |
| 14 | PA4 | GPIO_EXTI4 | POWER_MONITOR | 5.2.4.3.POWER_MONITOR |
| 15 | PA5 | GPIO_EXTI5 | MSW_ON | 5.2.3.4.MSW_ON |
| 18 | PB0 | GPIO_Output | IR_P_ON | 5.2.2.1.IR_P_ON |
| 19 | PB1 | GPIO_Output | ERP_SENSOR_ON | 5.2.2.3.ERP_SENSOR_ON |
| 20 | PB2 | GPIO_Output | /RESET | 5.2.2.4./RESET |
| 21 | PB10 | GPIO_Output | /USB2_OE | 5.2.2.8./USB2_OE |
| 26 | PB13 | GPIO_Output | MC_PWR_EN | 5.2.2.7.MC_PWR_EN |
| 29 | PA8 | GPIO_Input | MONI_24V11 | 5.2.1.2.MONI_24V11 |
| 30 | PA9* | USART1_TX |  |  |
| 31 | PA10* | USART1_RX |  |  |
| 32 | PA11 | GPIO_Input | SB_PG | 5.2.1.2.SB_PG |
| 34 | PA13 | SYS_SWDIO | TMS |  |
| 37 | PA14 | SYS_SWCLK |  |  |
| 38 | PA15 | GPIO_Input | MC3.3VON_MONI | 5.2.1.3.MC3.3VON_MONI |
| 39 | PB3 | GPIO_Output | SB_PWR_EN | 5.2.2.8.SB_PWR_EN |
| 41 | PB5 | GPIO_Output | MC_P_ON | 5.2.2.2.MC_P_ON |
| 42 | PB6 | I2C1_SCL |  |  |
| 43 | PB7 | I2C1_SDA |  |  |
| 45 | PB8 | GPIO_Output | /DISCHG | 5.2.2.6./DISCHG |
| 46 | PB9 | GPIO_Output | /RST_SLP2 | 5.2.2.11./RST_USB |

## Register specifications

Of the PS-CPURegister specificationsRefer to the datasheet in the references.

Since register specifications are provided for each peripheral,refer to those.

[References]

Reference 1: STM32F0x1_2_8RM(RM0091)_Japanese_rev1.pdf

# Software configuration

## Software configuration diagram

The PS-CPU is structured as two projects in order to perform IAP.

| Main project | The main project for the PS-CPU. Performs power control and extended IO control. |
| --- | --- |
| IAP project | Project for rewriting the Main project. Program that runs when a program rewrite command is received. |

[References]

## Source / folder list

PS-CPUThe folder structure of  is as follows.

| common | Wrapper for CPU-independent drivers. |
| --- | --- |
| iap | Project for PS-CPU IAP processing |
| main | Project for main PS-CPU processing |

PS-CPUThe list of source codeis shown below.

| subcpu |  |  |  |  |  |  |  |  | PS-CPU folder |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PowerSubCPU.uvmpw |  |  |  |  |  |  |  | PS-CPU project file |
|  | common |  |  |  |  |  |  |  | Driver wrapper related folder |
|  |  | Drivers |  |  |  |  |  |  | - |
|  |  |  | BSP |  |  |  |  |  | - |
|  |  |  |  | Inc |  |  |  |  | - |
|  |  |  |  |  | BSP_STM32F030_Nucleo.h |  |  |  | Header file for STM32F030 series evaluation board |
|  |  |  |  | Src |  |  |  |  | - |
|  |  |  |  |  | BSP_GPIO_STM32F030_Nucleo.c |  |  |  | For STM32F030 series evaluation boardSource file |
|  |  |  | CMSIS |  |  |  |  |  | - |
|  |  |  |  | Device |  |  |  |  | - |
|  |  |  |  |  | ST |  |  |  | - |
|  |  |  |  |  |  | STM32F0xx |  |  | - |
|  |  |  |  |  |  |  | Include |  | - |
|  |  |  |  |  |  |  |  | Driver_Common.h |  |
|  |  |  |  |  |  |  |  | FLASH_stm32f0xx.h | Header file for CMSIS interface flash driver |
|  |  |  |  |  |  |  |  | I2C_STM32F030.h | For CMSIS interfaceI2CDriver header file |
|  |  |  |  |  |  |  |  | I2C_stm32f0xx.h | For CMSIS interfaceI2CDriver header file |
|  |  |  |  |  |  |  | Source |  | - |
|  |  |  |  |  |  |  |  | FLASH_stm32f0xx.c | For CMSIS interface flash driverSourceFile |
|  |  |  |  |  |  |  |  | I2C_STM32F030.c | For CMSIS interfaceI2CFor driverSourceFile |
|  |  |  |  |  |  |  |  | I2C_stm32f0xx.c | For CMSIS interfaceI2CFor driverSourceFile |
|  |  |  | STM32F0xx_HAL_Driver |  |  |  |  |  | HAL driver shared by iap and main output by CubeMX (manually moved) |
|  |  |  |  | Inc |  |  |  |  |  |
|  |  |  |  |  |  |  |  | stm32f0xx_hal.h |  |
|  |  |  |  |  |  |  |  | … |  |
|  |  |  |  | Src |  |  |  |  |  |
|  |  |  |  |  |  |  |  | stm32f0xx_hal.c |  |
|  |  |  |  |  |  |  |  | … |  |
|  | iap |  |  |  |  |  |  |  |  |
|  |  | App |  |  |  |  |  |  |  |
|  |  |  | entry_iap.c |  |  |  |  |  | CubeMX generated file(originally main.c) |
|  |  |  | km_i2c_iap.c |  |  |  |  |  | KM-created I2C processing source file |
|  |  |  | km_extend_io_iap.c |  |  |  |  |  | KM extended IO processing(IAPon the sideMSW_ONprocessing) |
|  |  | Inc |  |  |  |  |  |  |  |
|  |  |  | entry_iap.h |  |  |  |  |  | Main processing system header |
|  |  |  | km_i2c_iap.h |  |  |  |  |  | KM I2C processing-related header |
|  |  |  | mxconstants.h |  |  |  |  |  | CubeMX generated file |
|  |  |  | km_ver.h |  |  |  |  |  | KM-created version information header file |
|  |  |  | stm32f0xx_hal_conf.h |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_it.h |  |  |  |  |  | CubeMX generated file |
|  |  | MDK-ARM |  |  |  |  |  |  |  |
|  |  |  | PowerSubCPU_iap.uvprojx |  |  |  |  |  | IAP project file |
|  |  |  | PowerSubCPU_iap |  |  |  |  |  | - |
|  |  |  | startup_stm32f030x8.s |  |  |  |  |  | CubeMX generated file |
|  |  | PowerSubCPU_iap.ioc |  |  |  |  |  |  | CubeMX for IAP |
|  |  | Src |  |  |  |  |  |  |  |
|  |  |  | mx_init.c |  |  |  |  |  | File that replaces the main.c generated by CubeMX |
|  |  |  | stm32f0xx_hal_msp.c |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_it.c |  |  |  |  |  | CubeMX generated file |
|  |  |  | system_stm32f0xx.c |  |  |  |  |  | CubeMX generated file |
|  | main |  |  |  |  |  |  |  |  |
|  |  | App |  |  |  |  |  |  |  |
|  |  |  | entry.c |  |  |  |  |  | Main processing |
|  |  |  | km_extend_io.c |  |  |  |  |  | KM extended I/O processing |
|  |  |  | km_i2c.c |  |  |  |  |  | KM-created I2C processing |
|  |  |  | km_it.c |  |  |  |  |  | KM-created interrupt processing |
|  |  |  | km_adc.c |  |  |  |  |  | KM ADC processing |
|  |  | Inc |  |  |  |  |  |  |  |
|  |  |  | entry.h |  |  |  |  |  | Main processing system header |
|  |  |  | km_extend_io.h |  |  |  |  |  | Extended I/O-related header |
|  |  |  | km_i2c.h |  |  |  |  |  | KM I2C processing-related header |
|  |  |  | km_adc.h |  |  |  |  |  | KM ADC processing-related header |
|  |  |  | km_it.h |  |  |  |  |  | KM-created interrupt processing-related header |
|  |  |  | km_ver.h |  |  |  |  |  | KM-created version information header file |
|  |  |  | mxconstants.h |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_hal_conf.h |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_it.h |  |  |  |  |  | CubeMX generated file |
|  |  | MDK-ARM |  |  |  |  |  |  |  |
|  |  |  | PowerSubCPU.uvprojx |  |  |  |  |  | Main project file |
|  |  |  | PowerSubCPU |  |  |  |  |  |  |
|  |  |  | startup_stm32f030x8.s |  |  |  |  |  |  |
|  |  | PowerSubCPU.ioc |  |  |  |  |  |  | CubeMX for main |
|  |  | Src |  |  |  |  |  |  |  |
|  |  |  | mx_init.c |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_hal_msp.c |  |  |  |  |  | CubeMX generated file |
|  |  |  | stm32f0xx_it.c |  |  |  |  |  | CubeMX generated file |

## Development environment

The development environment for the PS-CPU is described below.。

### STM32CubeMX

STM32CubeMX is a source code generation tool provided by STMicro.

Using a GUI, you can configure pin assignments and enable/disable peripherals, and based on those settings, projects for Keil and other environments are generated.

Use the following installer.

\\10.128.16.110\data1\29P.システム統合SoC開発\03_BSP\10.ツール\00.SubCPU_tool\STmicro\STM32Cube

SetupSTM32CubeMX-4.14.0.exe

Proceed with the installation by following the screens that appear.

※There are no special precautions.

7/25The version currently in use is Version 4.15.1, so start STM32CubeMX once andcheck the following.

Help -> Check for update

When you check this, Version 4.15.1 will be shown, so perform Install.

※Installand thenrestart the application; however, in some cases an error may occur at restart.If that happens,try running it in administrator mode.

#### Peripherals,PIN configuration method

Start STM32CubeMX, and click the Pinout tab to bring up the following screen.

To enable a peripheral,as in the red-framed area,select the item and check it to enable it.

For PIN configuration,as in the blue-framed area,select the PIN and choose which function to enable.

If you need to perform more detailed settings, click the Configuration tab and then click each peripheral’s button to configure the detailed settings.

#### Code generation

Once the settings for each peripheral are complete, click Project->Generate Code to generate the code.

Code is generated in the location where the *.ioc (STM32CubeMX project file) resides.

For the PS-CPU for the S800, STM32CubeMX projects are provided separately for the main project and the iap project, and code must be generated for each of them.

When you perform code generation, main.c is automatically generated, but for the PS-CPU for the S800,as is,it is not used; it must be merged with mx_init.c.

mThe reason main.c is not used as is, is that the main() functionneeds to be customized, and we basically do not want to modify the code generated by STM32CubeMX.

The main() function is defined in entry.c created by KM.

### μVision

μVision is an integrated development environment provided by ARM (Keil).

It supports the compiler, source control, and debugging using ulink.

Use the following installer.

\\10.128.16.110\data1\29U.次世代システム検証環境開発\01_共通\10.ツール\MDK-ARM

MDK515.EXE

Proceed with the installation by following the screens that appear.

※There are no special precautions.

#### Multi-project

For the PS-CPU for the S800, μVision is used to manage two projects (main and iap) within a single project.

In the top directory for source control,PowerSubCPU.uvmpwwhen you start it,a screen like the following appears, and you can see that it is divided into two projects.

To switch between two projects, right-click the project name (the one withattached).”Set as Active Project”will be displayed, so click it.

Then that project becomes active, and you can perform build, debug, etc.

#### Build

From μVision’s Project tab,	you can perform builds.

When using multi-projects, be careful which project is selected.

If you want to add build options,right-click the folder directly under each project (the project name withattached),and select the folder directly under it, then right-click it and choose”Options for Targetnamed – Target ‘Target name’”.

The following screen will appear; select the C/C++ tab. Add the desired build options to Preprocessor Symbols under Define:.

#### Debugger

To connect to the CPU, execute Debug->Start/Stop Debug Session.

When executed, it connects to the CPU and writes the binary files generated by each project to the flash ROM.

* When connecting to the CPU, loading to flash ROM is always performed.

By default, instead of stopping at the reset vector, execution advances to the start of main() and breaks there.

Debugging methods are the same as with a typical debuggerso it can be used intuitively.

If you only want to rewrite the flash ROM, execute Flash->Download. The CPU will not be debugged; only the data in ROM will be modified.

#### Adding files

To add a file, select the folder you want to add it to and right-click. Select “Add Existing File to Groupe folder name” and then choose the file you want to add.

Files added here are automatically included in the build.

# Control specifications

Control of the PS-CPU is performed based on the power sub-CPU control specifications.

[References]

Related document 2:Power Sub-CPU Control Specification (XX).docx

## SW control overview

### KM inputInput signal

The MFP signals used as inputs to the PS-CPU are as follows.

※ For interrupts and analog inputs, see a separate chapter.

| Pin type | Function | Signal name |
| --- | --- | --- |
| PA8 | GPIO_Input | MONI_24V11 |
| PA11 | GPIO_Input | SB_PG |
| PA15 | GPIO_Input | MC3.3VON_MONI |

#### MONI_24V11

Using a comparator, it is determined whether the DC24V11 power supply (engine) isat or above the specified value.

※ In the power supply Sub-CPU specification, the signal name is24V11_MONIbut for software development, since constant definitions cannot start with a numeral, the signal name has been changed.

MONI_24V11 When Low, discharge of the 24V is complete, and when High, discharge isnotcomplete.。

S800From MC_P_ON(IR_P_ON) Even if there is a request to set High, when discharge is not yet complete, suppress the output of MC_P_ON(IR_P_ON)and wait for discharge completion, then output MC_P_ON(IR_P_ON).

At startup, first wait for discharge to complete, then turn on power to the S800 500 ms after discharge completion.

#### SB_PG

S800 AONsection Power Good signal。

When SB_PG is Low, the AON section power is cut off.

When SB_PG is High, Power Good is being detected.

#### MC3.3VON_MONI

Power Good signal issued by the 3.3 V power supply for the mech-con block.

MC3.3VON_MONI LowWhen it changes to Low, wait for 0.5 seconds to elapse, then determine that the mech-con block power has been cut off.。

MC3.3VON_MONI When High,Power Good is being detected.

Control triggered by this signal is not performed by the PS-CPU.

If in the future it becomes necessary to control the mech-con block, it will be used.

### KM outputInput signal

The signals used as outputs from the PS-CPU are as follows.

| Pin type | Function | Signal name |
| --- | --- | --- |
| PB0 | GPIO_Output | IR_P_ON |
| PB1 | GPIO_Output | ERP_SENSOR_ON |
| PB2 | GPIO_Output | /RESET |
| PB3 | GPIO_Output | SB_PWR_EN |
| PB5 | GPIO_Output | MC_P_ON |
| PB8 | GPIO_Output | /DISCHG |
| PB9 | GPIO_Output | /RST_SLP2 |
| PB10 | GPIO_Output | /USB2_OE |
| PB13 | GPIO_Output | MC_PWR_EN |
| PC13 | GPIO_Output | WAKEUP |

#### IR_P_ON

Signal that controls the IR power relay.

At startup,simultaneously with /RESET,set IR_P_ON to High.

When POWER_MONITOR Low is detected, immediately set IR_P_ON to Low.

Except for the above two cases, control is performed according to the telegram from LPPP (2ndAddress 0x70: PB0).

However, when setting High,24V11_MONIwait for it to be Low(24 V discharge state) before setting High.

#### ERP_SENSOR_ON

Signal that cuts off power to the engine-related sensors (DOOR_OPEN, DEST_EDH, CLOSE).

In ErP lot.6 state and in SLEEP2 state with sensor off set, ERP_SENSOR_ON outputs Low, and otherwise outputs High.

Since the PS-CPU does not recognize the power-saving state, it makes the determination using the AP_PWR_EN signal.

AP_PWR_ENWhen this is Low and the telegram(2ndAddress 0x70: PB1)is Low, ERP_SENSOR_ON outputs Low.

AP_PWR_ENWhen this is Highorthe telegram(2ndAddress 0x70: PB1)is High,ERP_SENSOR_ON outputs High.

#### /RESET

Reset output signal to the S800 and peripheral circuits.

There are the following three control opportunities.

Immediately after PS-CPU startup, output /RESET Lowoutput

After MSW_ON goes High, output SB_PWR_EN High, then output /RESET High to release reset.

When /HRESET_REQ Low is detected, /RESET Low is output immediately.

#### SB_PWR_EN

Power control signal to the S800 AON section.

When MSW_ON High is detected,SB_PWR_EN High is output.

When /HRESET_REQ Low is detected, SB_PWR_EN Low is output.

However,as a common behavior for the above two triggers,MONI_24V11 in the case of High,MONI_24V11 it waits for Low (completion of 24V discharge),MONI_24V11 Lowand then outputs SB_PWR_EN High 0.5 seconds later.

#### MC_P_ON

MCuse(MechaCon block)Signal that controls the power relay.

At startup,simultaneously with /RESET,MCSet _P_ON to High.

When POWER_MONITOR Low is detected, immediatelyMCset _P_ON to Low.

Except for the above two cases, the telegram (2ndAddress 0x70: PB5).

However, when setting High,24V11_MONIwaits for Low (24V discharge state) and then sets it to High.

#### /DISCHG

No planned use

#### /RST_SLP2

Reset signal for AP peripheral components.

Since there is no external reset for the AP peripherals, the PS-CPU performs reset of the AP peripheral components.

Telegram (2ndAddress 0x70: PB5) Low orwhen AP_PWR_EN Low is detected, /RST_SLP2 Low is output.

Telegram (2ndAddress 0x70: PB5) When High andAP_PWR_EN High,/RST_USB High is output.

#### /USB2_OE

Specification undecided (T.B.D *To be described once the proxy response specification for wireless LAN is determined(9/Eplanned))

#### MC_PWR_EN

Power control signal for ICs around the MC (MechaCon block).

MONI_24V11 LowWhenis detected, MC_PWR_EN Low is output。

MC_PWR_EN High is output simultaneously with MC_P_ON High.

#### WAKEUP

Startup signal from PS-CPU to S800 AP issued by the PS-CPU.

When an interrupt is received from the RTC built into the PS-CPU, WAKEUP High is output.

Since this signal is directly connected to the RTC Alarm interrupt built into the PS-CPU, PS-CPU software does not intervene.

The S800 side shall directly control the RTC.

*Other Wakeup interrupts in S72/73 are controlled by LPPP.

### KMInterrupt Signals

The interrupt signals entering externally into the PS-CPU are as follows.

| Pin number | Pin type | Function | Signal name | Interrupt settings |
| --- | --- | --- | --- | --- |
| 11 | PA1 | GPIO_EXTI1 | /HRESET_REQ | External Interrupt Mode with Falling edge trigger detection |
| 12 | PA2 | GPIO_EXTI2 | AP_PWR_EN | External Interrupt Mode with Rising/Falling edge |
| 14 | PA4 | GPIO_EXTI4 | POWER_MONITOR | External Interrupt Mode with Falling edge trigger detection |
| 15 | PA5 | GPIO_EXTI5 | MSW_ON | External Interrupt Mode with Rising/Falling edge |

#### /HRESET_REQ

Restart request signal issued by the S800.

When MSW_ON is High,/HRESET High → Lowat that time,/the restart sequence is started.

When MSW_ON is Low,when /HRESET goes High → Low,power cutoff processingis started.

/HRESET Low → When High, it is judged that the S800 has started up (the PS-CPU does nothing).

#### AP_PWR_EN

Signal issued by the LPPP to control the AP power supply.

On the PS-CPU, it is used for the Low/High timing of the ERP_SENSOR_ON signal.

For details, see5.2.2.2. ERP_SENSOR_ONfor reference

#### POWER_MONITOR

DC5V detection signal issued by the low-voltage power supply

When POWER_MONITOR goes Low → High, nothing is done.

POWER_MONITOR High → Lowat that time,Outputs MC_P_ON Low and IR_P_ON Low unconditionally.

#### MSW_ON

Signal indicating the state of the main power switch.

The LIVE power supply that powers the PS-CPU will not be cut off as long as commercial power is being supplied, even if the main power switch is turned off.

While the main power switch is turned off, the PS-CPU is in "Sleepmode" to reduce power consumption.

[1] When an interrupt is detected with MSW_ON=1

1. Cancel its own "Sleepmode".

2. Turn on the SB power supply for the S800. (SB_PWR_EN=1)

3. After a specified time, release the reset of the SB. (RESETn=1)

4. Start the engine relay and the 3.3 V for the MC.

(MC_P_ON=1, IR_P_ON=1, MC_PWR_EN=1)

[2] When an interrupt is detected with MSW_ON=0 and SB_PWR_EN=0 (S800 stopped/restarting)

1. Abort the restart process and shut off all power supplies except the LIVE power supply.

2. After confirming that the PG of all power supplies except LIVE has dropped, transition itself to "Sleepmode".

[3] When an interrupt is detected with MSW_ON=0 and SB_PWR_EN=1

1. Cut off the engine relay. (MC_P_ON-0 and IR_P_ON=0)

2. Wait for HRESET_REQn=0 from the S800.

3. When HRESET_REQn=0 is detected, shut off all power supplies except the LIVE power supply.

4. After confirming that the PG of all power supplies except LIVE has dropped, transition itself to "Sleepmode".

### I2C control

Communication with the CA72 or LPPP is performed using I2C.

Due to the nature of I2C communication,CA72, LPPP becomes the master,The PS-CPU side is treated as a slave device.

#### Communication specifications

The communication specifications of the PS-CPU are as follows.

| I2C bus mode | Fast Mode (400KHz) |
| --- | --- |
| Address width | 7 bit(excluding R/Wbit)) |
| Slave address | 0x3D (RTC) |
|  | 0x3E(excluding RTC)) |

In I2C communication, the master side (CA72, LPPP) always sends the slave address of the device (PS-CPU)2ndaddressand then performs data write or read.

In I2C communication, following the 8-bit2ndaddresscomesthe following specifications apply.

・When writing to the PS-CPU, write 4 bytes of data.

・When reading from the PS-CPU, read 2 bytes of data.

*However, the program data for a program rewrite request is 1024 bytes.

The reason for this specification is that in general drivers, block transfer is supported for Write, but in many cases block transfer is not supported for Read.

For details of READ and WRITE, refer to the Power Sub-CPU Control Specification.

※10.3.1. Writing from the main CPU

※10.3.2. Reading by the main CPU

[References]

Related document 2:Power Sub-CPU Control Specification (XX).docx

#### STM32CubeMX settings

Set STM32CubeMX as shown in the following figure.

#### Control sequence

The control sequence of the PS-CPU is shown below.

Data reception is performed by interrupt; until an interrupt occurs, only the interrupt flag is checked, so that the specification minimizes the impact on port control.

As described in the I2C communication specifications of the Power Sub-CPU Control Specification, the master (S800) always writes the 2nd address,Writerequest is the starting point.

When writing to the 2nd address, communication is performed with 2nd address (1 byte) + write data (4 bytes) + Stop bit.

When reading from the 2nd address, after 2nd address (1 byte) + Stop bit, the slave side (PS-CPU) sends read data + Stop bit.

[References]

Related document 2:Power Sub-CPU Control Specification (XX).docx

#### PS-CPU 2ndaddressSpecifications

##### 5.1.5.3.1. RTC

List of registers that control the RTC.

*Same as the RTC register configuration of the STM32F031

| 2nd | 0x00 | 0x04 | 0x08 | 0x0C | 0x10 | 0x14 | 0x1C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Usage | RTC Time | RTC Date | RTC Control | RTC Initialization and Status | RTC Pre-scaler | RTC Wakeup Timer | RTC Alarm A |
| 0 | SU[3:0] | DU[3:0] | WUCKSEL[2:0] | ALRAWF | PREDIV_S[14:0] | WUT[15:0] | SU[3:0] |
| 1 |  |  |  | ALRBWF |  |  |  |
| 2 |  |  |  | WUTWF |  |  |  |
| 3 |  |  | TSEDGE | SHPF |  |  |  |
| 4 | ST[2:0] | DT[1:0] | REFCKON | INTS |  |  | ST[2:0] |
| 5 |  |  | BYPSHAD | RSF |  |  |  |
| 6 |  |  | FMT | INTF |  |  |  |
| 7 |  |  |  | INT |  |  | MASK1 |
| 8 | MNU[3:0] | MU[3:0] | ALRAE | ALRAF |  |  | MNU[3:0] |
| 9 |  |  |  | WUPF |  |  |  |
| 10 |  |  | WUTE |  |  |  |  |
| 11 |  |  | TSE | TSF |  |  |  |
| 12 | MNT[2:0] | MT | ALRAIE | TSOVF |  |  | MNT[2:0] |
| 13 |  | WDU[2:0] |  | TAMP1F |  |  |  |
| 14 |  |  | WUTIE | TAMP2F |  |  |  |
| 15 |  |  | TSIE | TAMP3F |  |  | MASK2 |
| 16 | HU[3:0] | YU[3:0] | ADD1H | RECALPF | PREDIV_A[6:0] |  | HU[3:0] |
| 17 |  |  | SUB1H |  |  |  |  |
| 18 |  |  | BKP |  |  |  |  |
| 19 |  |  | COSEL |  |  |  |  |
| 20 | HT[1:0] | YT[3:0] | POL |  |  |  | HT[1:0] |
| 21 |  |  | OSEL[1:0] |  |  |  |  |
| 22 | PM |  |  |  |  |  | PM |
| 23 |  |  | COE |  |  |  | MASK3 |
| 24 |  |  |  |  |  |  | DU[3:0] |
| 25 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  | DT[1:0] |
| 29 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  | WDSEL |
| 31 |  |  |  |  |  |  | MASL4 |

| 2nd | 0x24 | 0x28 | 0x2C | 0x30 | 0x34 | 0x38 | 0x3C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Usage | RTC Write Protection | RTC Sub Second | RTC Shiftt Control | RTC Timestamp Time | RTC Timestamp Date | RTC Timestamp Sub-second | RTC Calibration |
| 0 | KEY[7:0] | SS[15:0] | SUBFS[14:0] | SU[3:0] | DU[3:0] | SS[15:0] | CALM[8:0] |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  | ST[2:0] | DT[1:0] |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  | MNU[3:0] | MU[3:0] |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |
| 12 |  |  |  | MNT[2:0] | MT |  |  |
| 13 |  |  |  |  | WDU[2:0] |  | CALW16 |
| 14 |  |  |  |  |  |  | CALW8 |
| 15 |  |  |  |  |  |  | CALP |
| 16 |  |  |  | HU[3:0] |  |  |  |
| 17 |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |
| 20 |  |  |  | HT[1:0] |  |  |  |
| 21 |  |  |  |  |  |  |  |
| 22 |  |  |  | PM |  |  |  |
| 23 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |

##### 5.1.5.3.2. Internal Status

| 2nd | 0x68 | 0x6C |
| --- | --- | --- |
| Usage | Internal Status | Reserved |
| 0 | Startup Status 0: Cold boot 1: Reboot |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |
| 18 |  |  |
| 19 |  |  |
| 20 |  |  |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |

5.1.5.3.3 I/O Expansion

Signals required for I/O in MFP.

| 2nd | 0x70 |  | 0x74 |  | 0x78 |  | 0x7c |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Usage | Output expansionRead register ※Read Only |  | Output expansion bit High register ※Write Only |  | Output expansion bit Low register ※Write Only |  | Reserved |
| 0 | PB0 | IR_P_ON | PB0 | IR_P_ON | PB0 | IR_P_ON |  |
| 1 | PB1 | /ERP(sensor on) | PB1 | /ERP(sensor on) | PB1 | /ERP(sensor on) |  |
| 2 | PB2 | /RESET | PB2 | /RESET | PB2 | /RESET |  |
| 3 | PB3 | - | PB3 | - | PB3 | - |  |
| 4 | PB4 | - | PB4 | - | PB4 | - |  |
| 5 | PB5 | MC_P_ON | PB5 | MC_P_ON | PB5 | MC_P_ON |  |
| 6 | PB6 | - | PB6 | - | PB6 | - |  |
| 7 | PB7 | - | PB7 | - | PB7 | - |  |
| 8 | PB8 | - | PB8 | - | PB8 | - |  |
| 9 | PB9 | /RST_SLP2 | PB9 | /RST_SLP2 | PB9 | /RST_SLP2 |  |
| 10 | PB10 | - | PB10 | /ENG_STATUS1 | PB10 | /ENG_STATUS1 |  |
| 11 | PB11 | - | PB11 | /ENG_STATUS2 | PB11 | /ENG_STATUS2 |  |
| 12 | PB12 | - | PB12 | - | PB12 | - |  |
| 13 | PB13 | - | PB13 | - | PB13 | - |  |
| 14 | PB14 | - | PB14 | - | PB14 | - |  |
| 15 | PB15 | - | PB15 | - | PB15 | - |  |
| 16 | PB16 |  | PB16 |  | PB16 |  |  |
| 17 | PB17 |  | PB17 |  | PB17 |  |  |
| 18 | PB18 |  | PB18 |  | PB18 |  |  |
| 19 | PB19 |  | PB19 |  | PB19 |  |  |
| 20 | PB20 |  | PB20 |  | PB20 |  |  |
| 21 | PB21 |  | PB21 |  | PB21 |  |  |
| 22 | PB22 |  | PB22 |  | PB22 |  |  |
| 23 | PB23 |  | PB23 |  | PB23 |  |  |
| 24 | PB24 |  | PB24 |  | PB24 |  |  |
| 25 | PB25 |  | PB25 |  | PB25 |  |  |
| 26 | PB26 |  | PB26 |  | PB26 |  |  |
| 27 | PB27 |  | PB27 |  | PB27 |  |  |
| 28 | PB28 |  | PB28 |  | PB28 |  |  |
| 29 | PB29 |  | PB29 |  | PB29 |  |  |
| 30 | PB30 |  | PB30 |  | PB30 |  |  |
| 31 | PB31 |  | PB31 |  | PB31 |  |  |

| 2nd | 0x80 |  | 0x84 | 0x88 | 0x8C |
| --- | --- | --- | --- | --- | --- |
| Usage | Input bit check |  | Reserved | Main checksum (2Byte) | Version check |
| 0 | PA0 | - |  | Checksum value (2Byte) | Minor[3:0] |
| 1 | PA1 | /HRESET_REQ |  |  |  |
| 2 | PA2 | AP_PWR_EN |  |  |  |
| 3 | PA3 | - |  |  |  |
| 4 | PA4 | POWER_MONITOR |  |  | Minor[7:4] |
| 5 | PA5 | - |  |  |  |
| 6 | PA6 | - |  |  |  |
| 7 | PA7 | MC_OE_REQ |  |  |  |
| 8 | PA8 | 24V11_MONI |  |  | Major[11:8] |
| 9 | PA9 | - |  |  |  |
| 10 | PA10 | MSW_ON |  |  |  |
| 11 | PA11 | - |  |  |  |
| 12 | PA12 | - |  |  | Major[15:12] |
| 13 | PA13 | - |  |  |  |
| 14 | PA14 | - |  |  |  |
| 15 | PA15 | MC3.3VON_MONI |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |

##### 5.1.5.3.4Program rewrite

Register for program rewriting.

| 2nd | 0x90 | 0x94 | 0x98 | 0x9C | 0xA0 |
| --- | --- | --- | --- | --- | --- |
| Usage | Program rewrite request | Program data rewrite completed | Program rewrite status | Reserved | Program rewrite data |
| 0 | Magic number: 0x11223344 Set this. If any data other than the above magic number is received, the program request will not be accepted |  | IAP transition status 0: Not completed 1: Completed |  | Program rewrite data is received as binary data in units of 1024 bytes. The data is received in order starting from the first address of the Flash ROM. |
| 1 |  |  | Program rewrite data status 0: Not completed 1: Completed |  |  |
| 2 |  | Program data completed 0: Not completed 1: Completed | Program data completion status 0: Not completed 1: Completed |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  | Checksum (1 byte) Value obtained by adding up the program data byte by byte |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  | Checksum error bit 0: No error occurred 1: Error occurred |  |  |
| 17 |  |  | PS-CPU internal error 0: No error occurred 1: Error occurred |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |

### IAP control

IAP control refers to control that rewrites the PS-CPU’s own program while the PS-CPU is running.

#### Basis of IAP control

The basis of IAP control is the IAP sample program provided by Hakuto.

The sample program is as follows.

< \\10.128.16.110\data1\29P.System Integration SoC Development\03_BSP\07.Outsourcing Management\Hakuto (PS-CPU)\From Hakuto\20160614_Question List, IAP Sample (STM32F030R8)>

IAP_Sample

The sample program is designed to obtain data via UART communication and change the program.

What is actually referenced is the project splitWhenThe concept and the process of transitioning from the main project to the iap project are referenced.

#### Project split

For the PS-CPU for S800, IAP control is implemented by splitting into two projects.

The project overview is as follows.

- Operate in accordance with the power Sub-CPU control specificationsmain project.

- main projectWheniap project that performs only the operations needed for rewriting.

Simple block diagram and operationA simple flow is shown below.

mThe main project starts up, andCA72or from LPPPIAPWhen a request is receivediapit jumps to the program.

iapThe programmreceives the main project’s program data, andmupdates the main project.

iapProjects can only be rewritten via ICE and cannot be rewritten from Subset.

#### FlashROMSegmentation

5.1.5.1. Project splittingAs described, by splitting into a main project and an iap project, it becomes necessary to divide the FlashROM where the program data is written.

The rough memory map of the STM32F031 is as follows.

Red framesectionis the area that stores the program.。

Since there is a 32KB area, the main projectuses 20KBand the iap projectuses 12KBeach.

※However, if the functional volume of each project increasesand the program size growsat any point,the area will beadjusted as needed.

The allocation is as follows.

#### I2C communication

Upon receiving a program rewrite requestTelegram (2ndaddress：0x90and data:0x11223344)it switches to IAP mode.

After switching to IAP mode, normal port processing is not performed.

Until switching to IAP, a program switching operation occurs, so a wait of about 5 ms is required.

After the program rewrite requestTelegram (2ndaddress：0xA0, data1024 bytes)is received, and the 1024 bytes are used to update the program area of the main project.

The data sent is binary data, on the assumption that data is sent sequentially starting from the first address of the flash ROM.

When reception of the program data from the main CPU is completed, the main CPU sets program rewrite completion(2ndaddress: 0x94, bit0=1)andalso sets the checksum value of the program data.

Wait for the program rewrite to finish, then the PS-CPU updates the program rewrite status(2ndaddress:0x98, bit0=1).

#### Program rewrite resetControl

When you run the IAP project and rewrite the program on the main side, you cannot run the rewritten main-side project unless a reset is applied.However,S800according to the specification, even if the main power (MSW_ON) is turned from ON to OFF, the PS-CPUremains running, and a reset cannot be applied unless the power plug is pulled.Therefore, the following measures are required.

・When the program rewrite is completed, perform a software reset by itself

・To control the reset timing, make it possible for the IAP side to detectMSW_ON

##### 5.1.5.5.1 MSW_ON support on the IAP side

STM32CubeMXStart up and modify the PIN settings on the IAP side so that GPIO MSW_ONcan be detected.

MainAssign MSW_ON to PA5 at the same location as on the main side

The GPIO Pin Configuration is set as follows.

Pin Name               : PA5

Signal on Pn           : n/a

GPIO output level      : n/a

GPIO mode              : External Interrupt Mode with Rising/Falling edge trigger detection

GPIO Pull-up/Pull-down : No pull-up and no pull-down

Maximum out speed      : n/a

Fast Mode              : n/a

User Label             : MSW_ON

Modified               : Check

Code generation by PIN settings ismainthe same as on the main side, so refer to4.3.1.2. Code generationSee .

##### 5.1.5.5.2 Software reset support

When the main power ON (MSW_ON=1) to main power OFF (MSW_ON=0) transition is detected while the IAP project is running, a software reset is performed on the device.

To execute a software reset, set the “SYSRESETREQ” bit in the Cortex-M0 Application Interrupt and Reset Control Register.This is achieved by setting it.

“Reset execution sequence”

- IAPWhile the  side program is running, main power ON (MSW_ON=1) ⇒ main power OFF (MSW_ON=0) is detected

- Program rewrite completion register (0x94 2ndbit) is already set

- When conditions 1. and 2. are both met,call the NVIC_SystemReset() function to perform a software reset

- When the software reset is completed, execution jumps to the main() function of the main project.

※Even if a main power ON/OFF is detected while rewriting has not yet completed,nothing in particular is done.

### RTC control

RTC is controlled by having the main CPU directly configure the RTC registers of the PS-CPU.

Therefore, the RTC registers are made accessible as I2C 2nd address range 0x00 to 0x5C.

For register details, refer to the STM32F031 datasheet.

[References]

Reference 1: STM32F0x1_2_8RM(RM0091)_Japanese_rev1.pdf

#### STM32CubeMX settings

Set STM32CubeMX as shown in the following figure.

If all Alarm settings are configured as mask settings, an Alarm interrupt occurs every clock cycle, which is not the intended behavior, so the automatically generated Alarm settings in mx_init.c are rewritten as follows.

sAlarm.AlarmDateWeekDay = 0;

By setting Weekday to 0, it becomes a non-existent weekday setting, and interrupts will no longer occur.

Set the above as the initial configuration, and have the actual Alarm settings performed from the main CPU side.

### First timeHandling of momentary power interruptions at power-on

When a normal momentary power interruption occurs, the sequence is: POWER_MONITOR goes down ⇒ wait for HRESET_REQ ⇒ power-off sequence ⇒ recovery when POWER_MONITOR comes back, and on the PS-CPU side, handling of the momentary interruption is done by using the HRESET_REQ issued by the AP side as the trigger.

However,When the power plug is insertedAt the first power-onThe AP side’s power is ONThere are cases where the AP side is not yet in the ON state and only the PS-CPU is awake. If a momentary interruption occurs in that state, HRESET_REQ cannot be issued, so the PS-CPU itself must perform the power-off process.

#### First timePowerONSequence

Below is a simple sequence for power-on.

As shown in the above sequence diagram, HRESET_REQ cannot be issued until LPPP sets AP_PWR_EN=1 and it is possible to confirm that LPPP has started up. During that time,(If a momentary interruption occurs while AP_PWR_EN=0)the PS-CPU itself must execute the power-off sequence.

Once AP_PWR_EN=1and LPPP becomes operational,after that, following HRESET_REQ poses no problem.

Also, when a momentary interruption occurs during DeepSleep/ERP, AP_PWR_EN becomes 0, but LPPP itself is already up and running, and even if a momentary interruption occurs in this state, HSince RESET_REQ can be issued,on the PS-CPU side, the sequence conforms to HRESET_REQ.

Taking into account handling of momentary interruptions at first power-on and processing of momentary interruptions during DeepSleep/ERP, the following shows the software sequence for the S800 PS-CPU when a momentary interruption occurs.

- PowerMonitor Down interrupt occurred (momentary power loss)

- “AP_PWR_EN status” and “DeepSleep/ERP or first power-on startup check flag(g_first_power_on)” determine the handling

AP_PWR_EN=0 and Flag OFF*: Go to power-off sequence (s800_power_off())

AP_PWR_EN=1 or  Flag ON *: Wait for HRESET_REQ

*Regarding the DeepSleep/ERP or first power-on startup check flag(g_first_power_on)About this flag

This flag is used becauseAP_PWR_ENalone cannot distinguish whether it is the first power-on startup orDeepSleep/ERP.

The usage is as follows。

・When AP_PWR_EN=1 turn this flag ON

・When the power-offsequence is executed,turn this flag OFF

⇒At the first power-on, the flag is OFF

⇒Regardless of AP_PWR_EN, during DeepSleep/ERP the flag is ON

- PowerMonitorWait for PowerMoni to recover and proceed to the power restoration sequence

### Checksum / Version information

[Checksum specification]

・MAdd up in order the 20KB of data from the main program at 0x08000000 to 0x08005000,and use the lower 16 bits of the total as the checksum.

・The checksum is calculated once at startup within the main() functiong_checksum_value and stored in it

・I2CVia 2nd address 0x88the checksum valuecan be obtainedispossible

※5.1.5.3.3 I/OSee expansion

[Version information specification]

・Modify the version information managed as 4 bytes to 2 bytes

Upper 8 bits -> Major version

Lower 8 bits -> Minor version

・　I2CVia 2nd address 0x8C can be obtained. Upper16bitwill be left unused

※5.1.5.3.3 I/OSee expansion

・4ByteThe interface and command "EXTEND_VERSION_COMMAND_2" that can obtain higher versions of the specification will be left as is

・MAINThe same specification shall be used for both the project and the IAP project

## Overview sequence diagram

The overview sequence diagram is shown below.

### Overall sequence diagram

The overall flow of the PS-CPU is shown below.

Once the PS-CPU starts up, it repeatedly runs an infinite loop, performing various signal processing and I2C reception processing, and refreshes the WDG timer at the end of the loop. If processing can no longer be performed at any point in the control flow, the WDT can no longer be refreshed and an automatic reset is executed.

### Power-on sequence diagram

The sequence diagram from PS-CPU power-on to turning on the power of the S800 is shown below.

After confirming discharge of the 24V and that MSW_ON is ON, the power of the S800 is turned ON.

### Power-off sequence diagram

The sequence diagram for turning the power of the S800 OFF is shown below.

The PS-CPU turns the power of the S800 OFF when HRESET_REQ and MSW_ON occur.

While MSW_ON is OFF, the PS-CPU transitions to Sleep mode.

## Function list

On the PS-CPU for the S800,Newly createdimplementedthe list of functions is shown.

For drivers supported by the Mikawa BSP, refer to the following specification document.

\\10.128.19.213\guest\29P.System IntegrationSoC Development\PS-CPU\Doc\Function Specifications

Function Specifications.docx

[References]

References3. Function Specifications.docx

### main project

main/App/entry.c

| Function Name | ProcessingOverview | Related Control | Remarks |
| --- | --- | --- | --- |
| main | MMain processing of the main project | 5.9.1.1. main | - |
| jump2iap | Processing to switch to the IAP project | 5.9.1.2. jump2iap | - |
| DeInit | MShutdown processing for drivers running in the main project | 5.9.1.3. DeInit | - |

main/App/km_extend_io.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| change_cmd2array | Convert the 2nd address to an index of an internal variable | 5.9.1.4. change_cmd2array | - |
| io_extend_read | Read from ports for extended I/O | 5.9.1.5. io_extend_read | - |
| io_extend_write | Write to ports for extended I/O | 5.9.1.6. io_extend_write | - |
| io_extend_memory_write | Store the state of ports for extended I/O in internal memory | 5.9.1.7. io_extend_memory_write | - |
| internal_sts_init | Initialize internal status | 5.9.1.8. internal_sts_init | - |
| mc3_3von_moni_proc | MC3.3VON_MONI signal processing | 5.9.1.9. mc3_3von_moni_proc | - |
| power_monitor_proc | POWER_MONITOR signal processing | 5.9.1.10. power_monitor_proc | - |
| hreset_req_proc | HRESET_REQ signal processing | 5.9.1.11. hreset_req_proc | - |
| extend_output_init | Output extension initialization | 5.9.1.12.extend_output_init |  |
| s800_power_off | S800 power-off processing | 5.9.1.13. s800_power_off | - |
| moni_24v_proc | 24V discharge confirmation processing | 5.9.1.14.moni_24v_proc |  |
| s800_power_on | S800 power-on processing | 5.9.1.15. s800_power_on | - |
| ap_power_en_proc | AP_PWR_EN signal processing | 5.9.1.16. ap_power_en_proc | - |
| erp_sensor_proc | ERP_SENSOR_ON signal processing | 5.9.1.17. erp_sensor_proc | - |
| rst_usb_proc | RST_USB signal processing | 5.9.1.18. rst_usb_proc | - |
| mc_p_on_proc | MC_P_ON signal processing | 5.9.1.19. mc_p_on_proc | - |
| ir_p_on_proc | IR_P_ON signal processing | 5.9.1.20. ir_p_on_proc | - |
| mc_oe_proc | MC_OE signal processing | 5.9.1.21. mc_oe_proc | - |
| mc_pwr_en_proc | MC_PWR_EN signal processing | 5.9.1.22. mc_pwr_en_proc | - |
| msw_on_proc | MSW_ON signal processing | 5.9.1.23.msw_on_proc |  |

main/App/km_i2c.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| rx_comp | II2C received data complete processing | 5.9.1.24. rx_comp | - |
| change_state | I2C slave send/receive switching processing | 5.9.1.25. change_state | - |
| i2c_recv_first | I2C initial receive processing | 5.9.1.26. i2c_recv_first | - |
| i2c_recv_wait | I2C receive processing | 5.9.1.27. i2c_recv_wait | - |
| checksum_calc | Calculate checksum of main program | 5.9.1.28. checksum_calc | - |

main/App/km_it.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| hreset_req_proc_handler | /HRESET_REQ handler | 5.9.1.29. hreset_req_proc_handler | - |
| km_exti_callback | External interrupt handler | 5.9.1.30. km_exti_callback | - |
| km_it_init | KM custom interrupt initialization processing | 5.9.1.31. km_it_init |  |
| HAL_TIM_PeriodElapsedCallback | Timer handler (1 ms period) | 5.9.1.32. HAL_TIM_PeriodElapsedCallback | - |
| km_timer_set | Timer setting | 5.9.1.33. km_timer_set | - |
| km_timer_get_state | Get timer status | 5.9.1.34. km_timer_get_state | - |

main/App/km_adc.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| adc_moni_5v_start_proc | 5V detection processing | 5.9.1.35. adc_moni_5v_start_proc | - |
| adc_moni_5v_off_proc | 5V drop detection processing | 5.9.1.36. adc_moni_5v_off_proc | - |

common/Drivers/BSP/Src/BSP_PWR_STM32F03x_Nucleo.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| BSP_PWR_enter_stopmode | STOP mode transition function | 5.9.1.37.BSP_PWR_enter_stopmode | - |
| BSP_PWR_enter_sleepmode | Sleep mode transition function | 5.9.1.38.BSP_PWR_enter_sleepmode | - |

### iap project

iap/App/entry_iap.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| main | MMain processing of the main project | 5.9.2.1. main | - |

iap/App/km_i2c_iap.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| rx_comp | II2C received data complete processing | 5.9.2.2. rx_comp | - |
| change_state | I2C slave send/receive switching processing | 5.9.2.3. change_state | - |
| i2c_recv_first | I2C initial receive processing | 5.9.2.4. i2c_recv_first | - |
| i2c_recv_wait | I2C receive processing | 5.9.2.5. i2c_recv_wait | - |
| flash_write_32 | Main program flash write | 5.9.2.6. flash_write_32 | - |
| main_project_flash_erase | Main program flash erase function | 5.9.2.7. main_project_flash_erase | - |
| check_sum_calc | Main program data checksum verification function | 5.9.2.8. check_sum_calc | - |

iap/App/km_extend_io_iap.c

| Function Name | Processing Overview | Related Control | Remarks |
| --- | --- | --- | --- |
| msw_on_proc_iap | MSW_ONSignal processing | 5.9.2.9. msw_on_proc_iap | - |
| km_exti_callback_iap | External interrupt handlerProcessing | 5.9.2.10. km_exti_callback_iap | - |
| km_it_init_iap | Interrupt initialization processing | 5.9.2.11. km_it_init_iap | - |

## Constant specifications

On the PS-CPU for the S800,Newly addedList of constants to be added.

### main project

main/Inc/entry.h

| Constant name | Value | Contents |
| --- | --- | --- |
| IAP_DEFAULT_ADD | 0x08005000 | Flash ROM address where the IAP project is located |

main/Inc/km_extend_io.h

| Constant name | Value | Contents |
| --- | --- | --- |
| BIT_CALC(x) | 1 << x | Macro for bit shifting |
| OUTPUT_EXTEND_IR_P_ON_BIT | 0 | I2C communication 2nd address: 0x70 bit position of IR_P_ON |
| OUTPUT_EXTEND_MC_P_ON_BIT | 5 | I2C communication 2nd address: 0x70 bit position of MC_P_ON |
| OUTPUT_EXTEND_RESET_BIT | 2 | I2C communication 2nd address: 0x70 bit position of /RESET |
| OUTPUT_EXTEND_SB_PWR_EN_BIT | 3 | I2C communication 2nd address: 0x70 bit position of SB_PWR_EN |
| OUTPUT_EXTEND_WAKEUP_BIT | 0 | I2C communication 2nd address: 0x60 bit position of WAKEUP |
| OUTPUT_EXTEND_ERP_SENSOR_ON_BIT | 1 | I2C communication 2nd address: 0x70 bit position of ERP_SEMSOR_ON |
| OUTPUT_EXTEND_RST_SLP2_BIT | 9 | I2C communication 2nd address: 0x70 /RST_SLP2Bit position of |
| INPUT_EXTEND_24V11_MONI_BIT | 8 | I2C communication 2nd address: 0x74 bit position of 5V_MONI |
| INPUT_EXTEND_MC_3_3V_ON_MONI_BIT | 15 | I2C communication 2nd address: 0x74 bit position of MC_3.3V_ON_MONI |
| INPUT_EXTEND_POWER_MONITOR_BIT | 4 | I2C communication 2nd address: 0x74 bit position of POWER_MONITOR |
| INPUT_EXTEND_HRESET_REQ_BIT | 1 | I2C communication 2nd address: 0x74 bit position of HRESET_REQ |
| INPUT_EXTEND_AP_PWR_EN_BIT | 2 | I2C communication 2nd address: 0x74 bit position of AP_PWR_EN |
| INPUT_EXTEND_MSW_ON_BIT | 10 | I2C communication 2nd address: 0x74 MSW_ONBit position of |
| MC_P_ON_NUM | TYPE_Output_Extend0 | Index number where MC_P_ON is stored |
| IR_P_ON_NUM | TYPE_Output_Extend0 | Index number where IR_P_ON is stored |
| ERP_SENSOR_ON_NUM | TYPE_Output_Extend0 | Extended I/O number of ERP_SENSOR_ON |
| RST_SLP2_NUM | TYPE_Output_Extend0 | Extended I/O number of RST_SLP2 |
| STATE_INT | 0 | Call from interrupt |
| STATE_NORMAL | 1 | Call from normal state |
| POWER_ON_FLG_START | 0 | Power ON or reboot state |
| POWER_ON_FLG_NORMAL | 1 | State after startup |
| FIRST_POWER_ON_FLG_START | 0 | First power-on startup(for momentary power interruption) |
| FIRST_POWER_ON_FLG_NORMAL | 1 | First power-on startupis not(for momentary power interruption) |

main/Inc/km_i2c.h

| Constant name | Value | Contents |
| --- | --- | --- |
| COMMAND_RWC_BUF_SIZE | 0x10 | Read/Write command receive buffer size |
| COMMAND_TX_SIZE | 2 | Transfer size |
| READ_CMD_SIZE | 1 | Read command receive buffer size |
| SECOND_ADDR_ARRAY | 0x0 | 2ndAddress storage position |
| DATA_ARRAY | 0x1 | Data storage position |
| DATA_ARRAY_SIZE | 0x4 | Data size |
| RTC_TR_CMD | 0x0 | TR register 2ndAddress definition |
| RTC_TR_CMD_2 | 0x2 | TR register 2ndAddress definition |
| RTC_DR_CMD | 0x4 | DR register 2ndAddress definition |
| RTC_DR_CMD_2 | 0x6 | DR register 2ndAddress definition |
| RTC_CR_CMD | 0x8 | CR register 2ndAddress definition |
| RTC_CR_CMD_2 | 0xA | CR register 2ndAddress definition |
| RTC_ISR_CMD | 0xC | ISR register 2ndAddress definition |
| RTC_ISR_CMD_2 | 0xE | ISR register 2ndAddress definition |
| RTC_PRER_CMD | 0x10 | PRER register 2ndAddress definition |
| RTC_PRER_CMD_2 | 0x12 | PRER register 2ndAddress definition |
| RTC_WUTR_CMD | 0x14 | WUTR Register 2ndAddress definition |
| RTC_WUTR_CMD_2 | 0x16 | WUTR Register 2ndAddress definition |
| RTC_ALRMAR_CMD | 0x1C | ALRMAR Register 2ndAddress definition |
| RTC_ALRMAR_CMD_2 | 0x1E | ALRMAR Register 2ndAddress definition |
| RTC_WPR_CMD | 0x24 | WPR Register 2ndAddress definition |
| RTC_WPR_CMD_2 | 0x26 | WPR Register 2ndAddress definition |
| RTC_SSR_CMD | 0x28 | SSR Register 2ndAddress definition |
| RTC_SSR_CMD_2 | 0x2A | SSR Register 2ndAddress definition |
| RTC_SHIFTR_CMD | 0x2C | SHIFTR Register 2ndAddress definition |
| RTC_SHIFTR_CMD_2 | 0x2E | SHIFTR Register 2ndAddress definition |
| RTC_TSTR_CMD | 0x30 | TSTR Register 2ndAddress definition |
| RTC_TSTR_CMD_2 | 0x32 | TSTR Register 2ndAddress definition |
| RTC_TSDR_CMD | 0x34 | TSDR Register 2ndAddress definition |
| RTC_TSDR_CMD_2 | 0x36 | TSDR Register 2ndAddress definition |
| RTC_TSSSR_CMD | 0x38 | TSSSR Register 2ndAddress definition |
| RTC_TSSSR_CMD_2 | 0x3A | TSSR Register 2ndAddress definition |
| RTC_CALR_CMD | 0x3C | CALR Register 2ndAddress definition |
| RTC_CALR_CMD_2 | 0x3E | CALR Register 2ndAddress definition |
| RTC_TAFCR_CMD | 0x40 | TAFCR Register 2ndAddress definition |
| RTC_TAFCR_CMD_2 | 0x42 | TAFCR Register 2ndAddress definition |
| RTC_ALRMASSR_CMD | 0x44 | ALRMASSR Register 2ndAddress definition |
| RTC_ALRMASSR_CMD_2 | 0x46 | ALRMASSR Register 2ndAddress definition |
| RTC_BKP0R_CMD | 0x50 | BKPOR0Register 2ndAddress definition |
| RTC_BKP0R_CMD_2 | 0x52 | BKPOR0Register 2ndAddress definition |
| RTC_BKP1R_CMD | 0x54 | BKPOR1Register 2ndAddress definition |
| RTC_BKP1R_CMD_2 | 0x56 | BKPOR1Register 2ndAddress definition |
| RTC_BKP2R_CMD | 0x58 | BKPOR2Register 2ndAddress definition |
| RTC_BKP2R_CMD_2 | 0x5A | BKPOR2Register 2ndAddress definition |
| RTC_BKP3R_CMD | 0x5C | BKPOR3Register 2ndAddress definition |
| RTC_BKP3R_CMD_2 | 0x5E | BKPOR3Register 2ndAddress definition |
| INT_STS_COMMAND | 0x60 | Interrupt status 2ndAddress definition |
| INT_STS_COMMAND_2 | 0x62 | Interrupt status 2ndAddress definition |
| INTERNEAL_STS_COMMAND | 0x68 | Internal status 2ndAddress definition |
| INTERNEAL_STS_COMMAND_2 | 0x6E | Internal status 2ndAddress definition |
| INTERNEAL_STS_COMMAND_BOOT_STATES | 0x00000001 | Startup StatusBit position |
| INTERNEAL_STS_COMMAND_BOOT_STATES_COLD | 0x0 | Coldboot |
| INTERNEAL_STS_COMMAND_BOOT_STATES_REBOOT | 0x1 | Reboot |
| EXTEND_OUTPUT_READ_COMMAND | 0x70 | Extended outputRegister for Read 2ndAddress definition |
| EXTEND_OUTPUT_READ_COMMAND_2 | 0x72 | Extended outputRegister for Read 2ndAddress definition |
| EXTEND_OUTPUT_HIGH_COMMAND | 0x74 | Extended inputRegister for Bit High setting 2ndAddress definition |
| EXTEND_OUTPUT_HIGH_COMMAND_2 | 0x76 | ExtendedRegister for output Bit High setting 2ndAddress definition |
| EXTEND_OUTPUT_LOW_COMMAND | 0x78 | Extended output Bit Low setting register 2ndAddress definition |
| EXTEND_OUTPUT_LOW_COMMAND_2 | 0x7A | Extended output Bit Low setting register 2ndAddress definition |
| EXTEND_INPUT_COMMAND | 0x80 | Extended input register 2ndAddress definition |
| EXTEND_INPUT_COMMAND_2 | 0x82 | Extended input register 2ndAddress definition |
| EXTEND_IO_COMMAND_BASE | EXTEND_OUTPUT_COMMAND | Base address of extended IO commands |
| EXTEND_VERSION_COMMAND | 0x8C | Version information 2ndDefinition of addresses |
| EXTEND_VERSION_COMMAND_2 | 0x8E | Version information 2ndDefinition of addresses |
| IAP_PG_COMMAND | 0x90 | Program rewrite request2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_INFO | 0x94 | Program data rewrite completed2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_INFO_COMP | 0x00000004 | Program data rewrite complete Program data complete bit |
| IAP_PG_COMMAND_CHANGE_INFO_CHECKSUM | 0x0000FF00 | Program data rewrite complete Checksum bit |
| IAP_PG_COMMAND_CHANGE_INFO_CHECKSUM_SHIFT | 16 | Program data rewrite complete Checksum bit shift value |
| IAP_PG_COMMAND_CHANGE_STATUS | 0x98 | Program rewrite status2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_STATUS_IAP | 0x00000001 | Program rewrite status IAP transition status |
| IAP_PG_COMMAND_CHANGE_STATUS_DATA_CMD_STS | 0x00000002 | Program rewrite status Program rewrite data status |
| IAP_PG_COMMAND_CHANGE_STATUS_PG_CMD_STS | 0x00000004 | Program rewrite status Program data completion status |
| IAP_PG_COMMAND_CHANGE_STATUS_CHKSUM_ERR | 0x00010000 | Program rewrite status Checksum error |
| IAP_PG_COMMAND_CHANGE_STATUS_PSCPU_ERR | 0x00020000 | Program rewrite status PS-CPU internal error |
| IAP_PG_COMMAND_DATA | 0xA0 | Program rewrite data 2ndAddress definition |
| IAP_PG_COMMAND_OFFSET | IAP_PG_COMMAND | Program rewrite requestOffset |
| IAP_PG_COMMAND_NUM | 4 | Number of program rewrite commands |
| IAP_REG_INDEX_CALC(x) | ((x - IAP_PG_COMMAND_OFFSET)/sizeof(uint32_t)) | IAP register index calculation macro |
| IAP_PG_COMMAND_MAGICNUM | 0x11223344 | Magic number for program rewrite requests |
| RTC_WRITEPROTECT_DISABLE | See note *1 | Disable protection of RTC registers |
| RTC_WRITEPROTECT_ENABLE | See note 2 | Enable protection of RTC registers |
| MAIN_APPLICATION_ADDRESS | (uint32_t)0x08000000 | MAIN project start address |
| MAIN_APPLICATION_SIZE | (uint32_t)0x5000 | MAIN project size |

※1

#define RTC_WRITEPROTECT_DISABLE do{ \ __IO uint32_t *w_p_addr=(uint32_t*)(RTC_BASE + RTC_WRITEPROTECT_COMMAND); \ *w_p_addr = 0xCA; \ *w_p_addr = 0x53; \ }while(0) \

※2

#define RTC_WRITEPROTECT_ENABLE do{ \ uint32_t *w_p_addr=(uint32_t*)(RTC_BASE + RTC_WRITEPROTECT_COMMAND); \ *w_p_addr = 0xFF; \ }while(0)

main/Inc/km_it.h

| Constant name | Value | Contents |
| --- | --- | --- |
| KM_TIMER_MAX_TIME | 32768 | Maximum timer value 32.768s |

main/Inc/km_adc.h

| Constant name | Value | Contents |
| --- | --- | --- |
| MONI_5V_ADC_CH | ADC_CH_0 | 5V ADC channel |
| ADC_VAL_POWER_ON | 0xb84 | 4.75V(4.75 / 2 * (2^12 - 1) / 3.3 ≒ 0xb84) |
| ADC_VAL_POWER_OFF | 0xACA | 4.45V(4.45 / 2 * (2^12 - 1) / 3.3 ≒ 0xACA) |

main/Inc/km_ver.h

| Constant name | Value | Contents |
| --- | --- | --- |
| VERSION_TYPE |  | Version information |
| VERSION_TYPE_SHIFT_16BIT |  | Shift value for displaying version information in 16 bits |
| VERSION_TYPE_SHIFT_32BIT |  | Shift value for displaying version information in 32 bits |
| REVISION_TYPE |  | Revision information |
| REVISION_TYPE_SHIFT_16BIT |  | Shift value for displaying revision information in 16 bits |
| REVISION_TYPE_SHIFT_32BIT |  | Shift value for displaying revision information in 32 bits |
| MODEL_TYPE |  | Model information |
| MODEL_TYPE_SHIFT_16BIT |  | Shift value for displaying Model information in 16-bit |
| MODEL_TYPE_SHIFT_32BIT |  | Shift value for displaying Model information in 32-bit |
| VERSION_MAIN_U |  | Version information Upper (16-bit) |
| VERSION_MAIN_L |  | Version information Lower (16-bit) |
| VERSION_MAIN |  | Version information (32-bit) |
| VERSION_MAJOR | 0x01 | Major VersionInformation |
| VERSION_MINOR | 0x00 | Minor Version Information |
| VERSION_TYPE_SHIFT_16BIT | 8 | 16bit Version shift value |
| VERSION_MAIN_U | ((VERSION_MAJOR<< VERSION_TYPE_SHIFT_16BIT)¦ VERSION_MINOR) | Version information (16-bit) |
| VERSION_MAIN_L | 0 | Version information (unused) |
| VERSION_MAIN | VERSION_MAIN_U | Version information (16-bit) |

### iapProject

iap/Inc/km_i2c_iap.h

| Constant name | Value | Contents |
| --- | --- | --- |
| COMMAND_RWC_BUF_SIZE | 1025 | Read/Write command receive buffer size |
| COMMAND_TX_SIZE | 0x2 | Read command transmit buffer size |
| SECOND_ADDR_ARRAY | 0x0 | 2ndAddress storage position |
| DATA_ARRAY | 0x1 | Data storage position |
| DATA_ARRAY_SIZE | 0x4 | Data size |
| RTC_TR_CMD | 0x0 | TR register 2ndAddress definition |
| RTC_TR_CMD_2 | 0x2 | TR register 2ndAddress definition |
| RTC_DR_CMD | 0x4 | DR register 2ndAddress definition |
| RTC_DR_CMD_2 | 0x6 | DR register 2ndAddress definition |
| RTC_CR_CMD | 0x8 | CR register 2ndAddress definition |
| RTC_CR_CMD_2 | 0xA | CR register 2ndAddress definition |
| RTC_ISR_CMD | 0xC | ISR register 2ndAddress definition |
| RTC_ISR_CMD_2 | 0xE | ISR register 2ndAddress definition |
| RTC_PRER_CMD | 0x10 | PRER register 2ndAddress definition |
| RTC_PRER_CMD_2 | 0x12 | PRER register 2ndAddress definition |
| RTC_WUTR_CMD | 0x14 | WUTR Register 2ndAddress definition |
| RTC_WUTR_CMD_2 | 0x16 | WUTR Register 2ndAddress definition |
| RTC_ALRMAR_CMD | 0x1C | ALRMAR Register 2ndAddress definition |
| RTC_ALRMAR_CMD_2 | 0x1E | ALRMAR Register 2ndAddress definition |
| RTC_WPR_CMD | 0x24 | WPR Register 2ndAddress definition |
| RTC_WPR_CMD_2 | 0x26 | WPR Register 2ndAddress definition |
| RTC_SSR_CMD | 0x28 | SSR Register 2ndAddress definition |
| RTC_SSR_CMD_2 | 0x2A | SSR Register 2ndAddress definition |
| RTC_SHIFTR_CMD | 0x2C | SHIFTR Register 2ndAddress definition |
| RTC_SHIFTR_CMD_2 | 0x2E | SHIFTR Register 2ndAddress definition |
| RTC_TSTR_CMD | 0x30 | TSTR Register 2ndAddress definition |
| RTC_TSTR_CMD_2 | 0x32 | TSTR Register 2ndAddress definition |
| RTC_TSDR_CMD | 0x34 | TSDR Register 2ndAddress definition |
| RTC_TSDR_CMD_2 | 0x36 | TSDR Register 2ndAddress definition |
| RTC_TSSSR_CMD | 0x38 | TSSSR Register 2ndAddress definition |
| RTC_TSSSR_CMD_2 | 0x3A | TSSR Register 2ndAddress definition |
| RTC_CALR_CMD | 0x3C | CALR Register 2ndAddress definition |
| RTC_CALR_CMD_2 | 0x3E | CALR Register 2ndAddress definition |
| RTC_TAFCR_CMD | 0x40 | TAFCR Register 2ndAddress definition |
| RTC_TAFCR_CMD_2 | 0x42 | TAFCR Register 2ndAddress definition |
| RTC_ALRMASSR_CMD | 0x44 | ALRMASSR Register 2ndAddress definition |
| RTC_ALRMASSR_CMD_2 | 0x46 | ALRMASSR Register 2ndAddress definition |
| RTC_BKP0R_CMD | 0x50 | BKPOR0Register 2ndAddress definition |
| RTC_BKP0R_CMD_2 | 0x52 | BKPOR0Register 2ndAddress definition |
| RTC_BKP1R_CMD | 0x54 | BKPOR1Register 2ndAddress definition |
| RTC_BKP1R_CMD_2 | 0x56 | BKPOR1Register 2ndAddress definition |
| RTC_BKP2R_CMD | 0x58 | BKPOR2Register 2ndAddress definition |
| RTC_BKP2R_CMD_2 | 0x5A | BKPOR2Register 2ndAddress definition |
| RTC_BKP3R_CMD | 0x5C | BKPOR3Register 2ndAddress definition |
| RTC_BKP3R_CMD_2 | 0x5E | BKPOR3Register 2ndAddress definition |
| INT_STS_COMMAND | 0x60 | Interrupt status2ndAddress definition |
| INT_STS_COMMAND_2 | 0x62 | Interrupt status2ndAddress definition |
| INTERNEAL_STS_COMMAND | 0x68 | Internal status 2ndAddress definition |
| INTERNEAL_STS_COMMAND_2 | 0x6E | Internal status 2ndAddress definition |
| INTERNEAL_STS_COMMAND_BOOT_STATES | 0x00000001 | Startup StatusBit position |
| INTERNEAL_STS_COMMAND_BOOT_STATES_COLD | 0x0 | Coldboot |
| INTERNEAL_STS_COMMAND_BOOT_STATES_REBOOT | 0x1 | Reboot |
| EXTEND_OUTPUT_READ_COMMAND | 0x70 | Extended output Read register 2ndAddress definition |
| EXTEND_OUTPUT_READ_COMMAND_2 | 0x72 | Extended output Read register 2ndAddress definition |
| EXTEND_OUTPUT_HIGH_COMMAND | 0x74 | Extended input Bit High setting register 2ndAddress definition |
| EXTEND_OUTPUT_HIGH_COMMAND_2 | 0x76 | Extended output Bit High setting register 2ndAddress definition |
| EXTEND_OUTPUT_LOW_COMMAND | 0x78 | Extended output Bit Low setting register 2ndAddress definition |
| EXTEND_OUTPUT_LOW_COMMAND_2 | 0x7A | Extended output Bit Low setting register 2ndAddress definition |
| EXTEND_INPUT_COMMAND | 0x80 | Extended input register 2ndAddress definition |
| EXTEND_INPUT_COMMAND_2 | 0x82 | Extended input register 2ndAddress definition |
| EXTEND_IO_COMMAND_BASE | EXTEND_OUTPUT_COMMAND | Base address of extended IO commands |
| EXTEND_VERSION_COMMAND | 0x8C | Version information 2ndAddress definition |
| EXTEND_VERSION_COMMAND_2 | 0x8E | Version information 2ndAddress definition |
| IAP_COMMAND | 0x90 | Program rewrite request 2ndAddress definition |
| IAP_PG_COMMAND_2 | 0x92 | Program rewrite request 2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_INFO | 0x94 | Program data rewrite completed2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_INFO_2 | 0x96 | Program data rewrite completed2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_INFO_COMP | 0x00000004 | Program data rewrite complete Program data complete bit |
| IAP_PG_COMMAND_CHANGE_INFO_CHECKSUM | 0x0000FF00 | Program data rewrite complete Checksum bit |
| IAP_PG_COMMAND_CHANGE_INFO_CHECKSUM_SHIFT | 8 | Program data rewrite complete Checksum bit shift value |
| IAP_PG_COMMAND_CHANGE_STATUS | 0x98 | Program rewrite status2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_STATUS_2 | 0x9A | Program rewrite status2ndAddress definition |
| IAP_PG_COMMAND_CHANGE_STATUS_IAP | 0x00000001 | Program rewrite status IAP transition status |
| IAP_PG_COMMAND_CHANGE_STATUS_DATA_CMD_STS | 0x00000002 | Program rewrite status Program rewrite data status |
| IAP_PG_COMMAND_CHANGE_STATUS_PG_CMD_STS | 0x00000004 | Program rewrite status Program data completion status |
| IAP_PG_COMMAND_CHANGE_STATUS_CHKSUM_ERR | 0x00010000 | Program rewrite status Checksum error |
| IAP_PG_COMMAND_CHANGE_STATUS_PSCPU_ERR | 0x00020000 | Program rewrite status PS-CPU internal error |
| IAP_PG_DATA_COMMAND | 0xA0 | 2ndDefinition of address: 0xA0 |
| IAP_PG_COMMAND_OFFSET | IAP_PG_COMMAND | Program rewrite request |
| IAP_PG_COMMAND_NUM | 4 | Number of program rewrite commands |
| IAP_REG_INDEX_CALC(x) | ((x - IAP_PG_COMMAND_OFFSET)/sizeof(uint32_t)) | IAP register index calculation macro |
| IAP_PG_COMMAND_MAGICNUM | 0x11223344 | Magic number for program rewrite requests |
| RTC_WRITEPROTECT_DISABLE | See note *1 | Disable protection of RTC registers |
| RTC_WRITEPROTECT_ENABLE | See note 2 | Enable protection of RTC registers |

※1

#define RTC_WRITEPROTECT_DISABLE do{ \ __IO uint32_t *w_p_addr=(uint32_t*)(RTC_BASE + RTC_WRITEPROTECT_COMMAND); \ *w_p_addr = 0xCA; \ *w_p_addr = 0x53; \ }while(0) \

※2

#define RTC_WRITEPROTECT_ENABLE do{ \ uint32_t *w_p_addr=(uint32_t*)(RTC_BASE + RTC_WRITEPROTECT_COMMAND); \ *w_p_addr = 0xFF; \ }while(0)

iap/App/km_i2c_iap.c

| Constant name | Value | Contents |
| --- | --- | --- |
| MAIN_APPLICATION_ADDRESS | (uint32_t)0x08000000 | Start address of the flash ROM where the Main project is stored |
| MAIN_APPLICATION_SIZE | (uint32_t)0x4000 | Size of the flash ROM where the Main project is stored |

## Enumeration typeSpecifications

On the PS-CPU for the S800,Newly addEnumeration typelist of .

### main project

main/Inc/km_extend_io.h

| Enumeration type name | TYPE_Io_Extend_Num |  |
| --- | --- | --- |
| Constant name | Value | Contents |
| TYPE_Output_Extend0 | 0 | Output extension number |
| TYPE_Input_Extend0 | 1 | Input extension number |
| TYPE_Int_Sts0 | 2 | Interrupt status number |
| TYPE_Internal_Sts0 | 3 | Internal status number |
| TYPE_Io_Extend_Max | 4 | Maximum extended I/O number |

main/App/km_i2c.c

| Enumeration type name | - |  |
| --- | --- | --- |
| Constant name | Value | Contents |
| TYPE_State_Init | 0 | I2C status initialization state |
| TYPE_State_Wait_Write | 1 | I2C status Write state |
| TYPE_State_Wait_Read | 2 | I2C status Read state |

main/Inc/km_it.h

| Enumeration type name | TYPE_Km_Timer_State- |  |
| --- | --- | --- |
| Constant name | Value | Contents |
| TYPE_Km_Timer_Normal | 0 | TimerStatus initialization state |
| TYPE_Km_Timer_Start | 1 | TimerStatusStartState |
| TYPE_Km_Timer_End | 2 | Timer status endState |
| TYPE_KM_Timer_Max | 3 | Maximum timer status value |

main/Inc/km_it.h

| Enumeration type name | TYPE_Km_Timer_Kind |  |
| --- | --- | --- |
| Constant name | Value | Contents |
| TYPE_Km_Timer_Rstusb | 0 | Timer for /RST_USB |
| TYPE_Km_Timer_MONI_24V11 | 1 | 24V discharge check timer |
| TYPE_Km_Timer_Kind_Max | 2 | Maximum timer type value |

### iap project

iap/App/km_i2c_iap.c

| Enumeration type name | - |  |
| --- | --- | --- |
| Constant name | Value | Contents |
| TYPE_State_Init | 0 | I2C status initialization state |
| TYPE_State_Wait_Write | 1 | I2C status Write state |
| TYPE_State_Wait_Read | 2 | I2C status Read state |

## Structure specification

On the PS-CPU for the S800,Newly addStructurelist of .

### main project

main/Inc/km_extend_io.h

| Structure name | km_io |  |
| --- | --- | --- |
| Member name | Type | Contents |
| extend_num | uint32_t | Extended I/O number |
| bit | uint32_t | Extended I/O bit position |
| port | GPIO_TypeDef * | Extended I/O port type |
| pin | uint16_t | Extended I/O pin position |

main/App/km_i2c.c

| Structure name | __i2c_info |  |
| --- | --- | --- |
| Member name | Type | Contents |
| state | int | Read/Write status |
| rx_buf[COMMAND_RWC_BUF_SIZE] | uint8_t | Receive buffer |
| second_addr | uint8_t | Second address |
| write_comp | int | Write completion flag |
| read_comp | int | Read completion flag |
| xfer_size | int | Transfer size |

main/Inc/km_it.h

| Structure name | km_timer_st |  |
| --- | --- | --- |
| Member name | Type | Contents |
| state | TYPE_Km_Timer_State | Timer status |
| ms_time | MillsecType | ms timer value (MAX=32768ms) |

### iapProject

iap/App/km_i2c_iap.c

| Structure name | __i2c_info |  |
| --- | --- | --- |
| Member name | Type | Contents |
| state | int | Read/Write status |
| rx_buf[COMMAND_RWC_BUF_SIZE] | uint8_t | Receive buffer |
| second_addr | uint8_t | Second address |
| write_comp | int | Write completion flag |
| read_comp | int | Read completion flag |
| xfer_size | int | Transfer size |

## Global variables

On the PS-CPU for the S800,Newly addGlobal variableslist of .

### main project

iap/App/km_extend_io.c

uint32_t g_io_extend_memory[TYPE_Io_Extend_Max];

Variable for storing extended I/O contents in memory.

iap/App/km_extend_io.c

int32_t g_power_on_flg = POWER_ON_FLG_START;

Flag that holds the startup state。

iap/App/km_extend_io.c

int32_t g_24v_check_flg = STATE_NONACTIVE;

Flag indicating whether 24V discharge needs to be checked。

iap/App/km_extend_io.c

g_first_power_on = FIRST_POWER_ON_FLG_START;

Flag to determine whether the first power-on startup is for momentary power interruption checking

main/App/km_i2c.c

struct __i2c_info i2c_info;

Manages the I2C stateVariable.

main/App/km_i2c.c

uint32_t tx_data=0;

I2C transmit buffer.

main/App/km_i2c.c

uint16_t g_checksum_value;

Checksum.

### iapProject

iap/App/km_i2c_iap.c

struct __i2c_info i2c_info;

Variable that manages the I2C state.

iap/App/km_i2c_iap.c

static uint32_t dest_addr;

Flash address to be rewritten.

iap/App/km_i2c_iap.c

uint32_t g_iap_reg[IAP_PG_COMMAND_NUM];

IAPI2C registers.

## List of const definitions

On the PS-CPU for the S800,Newly addList of const definitionsShows the list。

### main project

iap/App/km_extend_io.c

| Const definition name | const km_io io_extend[] |  |  |
| --- | --- | --- | --- |
| Extended I/O number | Extended I/O bit position | Extended I/O port type | Extended I/O pin position |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_IR_P_ON_BIT | IR_P_ON_GPIO_Port | IR_P_ON_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_MC_P_ON_BIT | MC_P_ON_GPIO_Port | MC_P_ON_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_RESET_BIT | _RESET_GPIO_Port | _RESET_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_SB_PWR_EN_BIT | SB_PWR_EN_GPIO_Port | SB_PWR_EN_Pin |
| TYPE_Int_Sts0 | OUTPUT_EXTEND_WAKEUP_BIT | WAKEUP_GPIO_Port | WAKEUP_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_ERP_SENSOR_ON_BIT | ERP_SENSOR_ON_GPIO_Port | ERP_SENSOR_ON_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_RST_USB_BIT | _RST_USB_GPIO_Port | _RST_USB_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_ENG_STATUS1_BIT | _ENG_STATUS1_GPIO_Port | _ENG_STATUS1_Pin |
| TYPE_Output_Extend0 | OUTPUT_EXTEND_ENG_STATUS2_BIT | _ENG_STATUS2_GPIO_Port | _ENG_STATUS2_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_5V_MONI_BIT | MONI_5V_GPIO_Port | MONI_5V_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_24V11_MONI_BIT | MONI_24V11_GPIO_Port | MONI_24V11_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_MC_3_3V_ON_MONI_BIT | MC3_3VON_MONI_GPIO_Port | MC3_3VON_MONI_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_POWER_MONITOR_BIT | POWER_MONITOR_GPIO_Port | POWER_MONITOR_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_HRESET_REQ_BIT | _HRESET_REQ_GPIO_Port | _HRESET_REQ_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_AP_PWR_EN_BIT | AP_PWR_EN_GPIO_Port | AP_PWR_EN_Pin |
| TYPE_Input_Extend0 | INPUT_EXTEND_MC_OE_REQ_BIT | MC_OE_REQ_GPIO_Port | MC_OE_REQ_Pin |

## Function specifications

5.2. Functionlistas shown infunctionsDetailed specifications of the functions are described。

### main project

#### main

[File]

main/App/entry.c

[Calling format]

int32_t main(void)

[Overview]

Mmain function

[Arguments]

None

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int32_t | 0 | A return value is set, but basically it never returns from main |

[Processing details]

- Enable the clock for the power control unit

- Board-specific initialization processing (MX_Init()) is called。

- Start the WDT

- Initialize internal status (internal_sts_init)

- Perform KM-specific interrupt initialization (km_it_init)

- Start the timer (TIM3started with a 1 ms period)

- Save the I/O state in memory (io_extend_memory_write())

- Set S800 to reset state (/RESET = 0)

- Check the 24V discharge state

  - Discharge complete

    - Turn ON power to S800 (s800_power_on())

  - Discharge not complete

9.2.1. Enable 24V discharge check flag

- I2C receive initialization processing (i2c_recv_first())

- MSW_ON signal processing (msw_on_proc())

- POWER_MONITOR signal processing (power_monitor_proc())

- 24V discharge confirmation processing (moni_24v_proc())

- MC_P_ON signal processing(mc_p_on_proc())

- IR_P_ON signal processing(ir_p_on_proc())

- MC3.3V signal processing(mc3_3von_moni_proc())

- MC_PWR_EN signal processing(mc_pwr_en_proc())

- /HRESET_REQ signal processing (hreset_req_proc())

- /RST_USB signal processing (rst_usb_proc())

- I2C receive processing (i2c_recv_wait())

- Refresh the watchdog timer (BSP_WDT_Refresh())

- 11Return to .

#### jump2iap

[File]

main/App/entry.c

[Calling format]

void jump2iap(void)

[Overview]

Jump to the IAP project

[Arguments]

None

[Return value]

None

[Processing details]

- Store the IAP jump destination address in a function pointer

* The IAP jump destination is the start address of the Flash ROM where the IAP is stored + 4

- IAP is stored inFlash ROMStore the stack pointer at the start address inthe sp register

- Jump to the destination set in 1.

#### DeInit

[File]

main/App/entry.c

[Calling format]

void DeInit(void)

[Overview]

Perform collective shutdown processing for each driver

[Arguments]

None

[Return value]

None

[Processing details]

- Timer shutdown processing (TIM3)

- Call the I2C shutdown processing

- RTCCall the shutdown processing

#### change_cmd2array

[File]

main/App/km_extend_io.c

[Calling format]

static TYPE_Io_Extend_Num change_cmd2array(uint8_t addr)

[Overview]

Obtain the index of an internal variable from the I2C 2nd address

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint8_t | addr | I2C communication 2nd address |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| TYPE_Io_Extend_Num | Index | Index of internal variables (io_extend_memory[], internal_sts_memory[])is returned. In case of error,TYPE_Io_Extend_Maxis returned |

[Processing details]

- Check the 2nd address

  - EXTEND_OUTPUT_COMMAND

    - TYPE_Output_Extend0is returned

  - EXTEND_INPUT_COMMAND

    - TYPE_Input_Extend0is returned

  - INT_STS_COMMAND

    - TYPE_Int_Sts0is returned

  - INTERNEAL_STS_COMMAND

    - TYPE_Internal_Sts0is returned

  - Other than the above

    - TYPE_Io_Extend_Maxis returned

#### io_extend_read

[File]

main/App/km_extend_io.c

[Calling format]

uint32_t io_extend_read(uint8_t addr)

[Overview]

Read from ports for extended I/O

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint8_t | addr | I2C communication 2nd address |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| uint32_t | Read data | Read data for the specified extended I/O number. If the specified extended I/O number is out of range, 0 is returned. |

[Processing details]

- Obtain the index of the internal variable from the 2nd address and perform index error check

  - If the extended I/O number isTYPE_Io_Extend_Maxor greater

1.1.2. Return with the read data set to 0

  - Otherwise 1.1.

1.2.1. Go to 2.

- Check the index

  - TYPE_Output_Extend0, TYPE_Input_Extend0, TYPE_Int_Sts0In case of

    - io_extendObtain the data for the extended I/O number from [] and read it from the I/O port

    - Return the read data as the return value.

  - TYPE_Internal_Sts0In case of

    - io_extend_memoryReturn the index portion

  - In cases other than the above

    - Return with the read data set to 0

#### io_extend_write

[File]

main/App/km_extend_io.c

[Calling format]

void io_extend_write(uint8_t addr,uint32_t data)

[Overview]

Write to the port for extended I/O

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint32_t | addr | I2C communication 2nd address |
| I | uint32_t | data | Data to be written to the specified extended I/O numberdata. |

[Return value]

None

[Processing details]

- Obtain the index of the internal variable from the 2nd address and perform index error check

  - If the extended I/O number isTYPE_Io_Extend_Maxor greater

1.1.2.Finish the processing

  - Otherwise 1.1.

    - Go to 2.

- io_extendObtain the data for the extended I/O number from [] and temporarily store it in memory (io_extend_memoryHold data in []).

- Update the I/O port when not under the following conditions.

  - When changing the ERP_SENSOR port, execute the dedicated process (erp_sensor_proc())

  - MC_P_ONWhen changing the portdo not operate the I/O port at this point.

  - IR_P_ONWhen changing the portdo not operate the I/O port at this point.

  - SB_PWR_ENDo not allow the port to be changed.

- According to the write dataperform a write to the I/O port.

#### io_extend_memory_write

[File]

main/App/entry.c

[Calling format]

void io_extend_memory_write(void)

[Overview]

ExtendedStore IO values in memory

[Arguments]

None

[Return value]

None

[Processing details]

- io_extendObtain the extended I/O number data from [] and, in memory (io_extend_memory[]), hold the data in the I2C telegram format.

#### internal_sts_init

[File]

main/App/entry.c

[Calling format]

void internal_sts_init(void)

[Overview]

Initialize variables for internal status.

[Arguments]

None

[Return value]

None

[Processing details]

- io_extend_memory[TYPE_Internal_Sts0]Initialize to 0

#### mc3_3von_moni_proc

[File]

main/App/km_extend_io.c

[Calling format]

void mc3_3von_moni_proc(void)

[Overview]

MC3.3VON_MONI signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- Do nothing

* As of 2017/5/1, there is no control using MC3.3VON_MONI

#### power_monitor_proc

[File]

main/App/km_extend_io.c

[Calling format]

void power_monitor_proc(uint8_t state)

[Overview]

POWER_MONITOR signal processing

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint8_t | state | State of the caller STATE_INT：Call from interrupt STATE_NORMAL：Normal statefromcall of |

[Return value]

None

[Processing details]

- Check state

  - STATE_INT

    - Set MC_P_ON to Low.

    - Set the internal memory information of MC_P_ON to Low.

    - Set IR_P_ON to Low.

    - Set the internal memory information of IR_P_ON to Low.

    - Check whether this is the first startup.(g_first_power_on) – First startup

      - S800turn off the power of (s800_power_off())

  - STATE_NORMAL

    - Restart state and POWER_MONITORis High and the 24-discharge check timer has expired

1.2.1.1. Turn ON the power of S800 (s800_power_on())

    - Other than 1.2.1.

      - Do nothing

#### hreset_req_proc

[File]

main/App/km_extend_io.c

[Calling format]

void hreset_req_proc(void)

[Overview]

HRESET_REQ signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- Turn OFF the power of S800 (s800_power_off())

- Set the power state flag toPOWER_ON_FLG_START

#### extend_output_init

[File]

main/App/km_extend_io.c

[Calling format]

void extend_output_init(void)

[Overview]

Output port initialization process

[Arguments]

None

[Return value]

None

[Processing details]

- Initialize the extended output port.

- InterruptStatus is initialized

- IR_P_ONSet to Low

- MC_P_ONSet to Low

- /RESETSet to Low

- SB_PWR_ENSet to Low

- ERP_SENSOR_ONSet to Low

- /RST_SLP2Set to Low

- /DISCHGSet to Low

- MC_PWR_ENSet to Low

#### s800_power_off

[File]

main/App/km_extend_io.c

[Calling format]

void s800_power_off(void)

[Overview]

S800 power-off processing

[Arguments]

None

[Return value]

None

[Processing details]

- Initial powerONStartup flagg_first_power_on to 0:Set to initial startup state

- Reset S800 (/RESETSet to Low).

- Turn OFF the power of S800 (SB_PWR_ENSet to Low).

- Set the reboot status of internal memory information to reboot

- Initialize output ports

- 24V dischargeEnable check flag。

#### moni_24v_proc

[File]

main/App/km_extend_io.c

[Calling format]

void moni_24v_proc(void)

[Overview]

24V discharge checkProcessing

[Arguments]

None

[Return value]

None

[Processing details]

- Check 24V discharge check flag

  - STATE_ACTIVE

    - MONI_24V11Check signal

      - Low

        - Set 24V monitoring timer to 500 ms

        - 24V check flag toSTATE_NONACTIVE

      - High

1.1.1.2.1. Do nothing

  - Otherwise 1.1.

    - Do nothing

#### s800_power_on

[File]

main/App/km_extend_io.c

[Calling format]

void s800_power_on(void)

[Overview]

S800 powerOnProcessing

[Arguments]

None

[Return value]

None

[Processing details]

- Check MSW_ON signal

  - High

    - Turn on S800 power (set SB_PWR_EN to High).

    - Release S800 reset (set /RESET to High).

    - Set startup state flag (POWER_ON_FLG_START) in power_on_flg.

  - Low

    - Do nothing

#### ap_power_en_proc

[File]

main/App/km_extend_io.c

[Calling format]

void ap_power_en_proc(void)

[Overview]

AP_PWR_EN signal processing

AP_PWR_ENWhenCalled from interrupt handler

[Arguments]

None

[Return value]

None

[Processing details]

- Control RST_USB signal processing (rst_usb_procCall )

- Initial powerONStartup flagg_first_power_on to 1:Set to not initial power startup

#### erp_sensor_proc

[File]

main/App/km_extend_io.c

[Calling format]

void erp_sensor_proc(void)

[Overview]

ERP_SENSOR_ON signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- AP_PWR_ENSignal ofand telegram informationCheck

  - AP_PWR_ENisLow and ERP_SENSOR in telegram is LowIn case of

    - ERP_SENSOR_ONLow

  - Other than 1.1

    - ERP_SENSOR_ONSet to High

#### rst_usb_proc

[File]

main/App/km_extend_io.c

[Calling format]

void rst_usb_proc(uint8_t state)

[Overview]

RST_USBSignal processing

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint8_t | state | State of the caller STATE_INT：Call from interrupt STATE_NORMAL：Normal statefromcall of |

[Return value]

None

[Processing details]

- Check caller state

  - Call from interrupt

    - Check AP_PWR_EN signal

      - If AP_PWR_EN is Low

        - Set RST_USB to Low

      - If AP_PWR_EN is High

        - Do nothing

  - Call from normal state

    - Check AP_PWR_EN and RST_SLP2 in telegram

      - AP_PWR_EN is High and the RST_SLP2 field in the frame is High

1.2.1.1.1. RST_USB toHighset to

      - Other than 1.2.1.1.

        - RST_USBto Lowset to

#### mc_p_on_proc

[File]

main/App/km_extend_io.c

[Calling format]

void mc_p_on_proc(void)

[Overview]

MC_P_ON signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- Startup state (power_on_flg) check

  - power_on_flgisPOWER_ON_FLG_NORMALIn case of

    - Check the value stored in memory (io_extend_memory)MC_P_ONCheck the value of

      - If High

        - MONI_24V11The value ofandAP_PWR_EN, POWER_MONITORThe value ofCheck

            - MONI_24V11isLowandAP_PWR_ENWhen this is Highand POWER_MONITOR is HighIn case of

              - Set MC_P_ON to High

            - Conditions other than the aboveIn case of

              - Set MC_P_ON to Low

      - If Low

1.1.1.2.1.Set MC_P_ON to Low.

  - power_on_flgisPOWER_ON_FLG_NORMALIn cases other than the above

    - AP_PWR_EN andMONI_24V11, POWER_MONITORCheck the value of

      - AP_PWR_EN is High andMONI_24V11isLowand POWER_MONITORwhen it is High

        - Set MC_P_ON to High

        - Set the internal memory information of MC_P_ON to High

        - Set IR_P_ON to High

        - Set the internal memory information of IR_P_ON to High

        - Set the startup condition flag (power_on_flg) toPOWER_ON_FLG_NORMAL.

      - When conditions other than 1.2.1.1. apply

        - Set IR_P_ON to Low

        - Set the internal memory information of IR_P_ON to Low.

        - Set MC_P_ON to Low

        - Set the internal memory information of MC_P_ON to Low.

#### ir_p_on_proc

[File]

main/App/km_extend_io.c

[Calling format]

void ir_p_on_proc(void)

[Overview]

IR_P_ON signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- Check the value stored in memory (io_extend_memory) the IR stored in_P_ONCheck the value of

  - If High

    - AP_PWR_EN andMONI_24V11and POWER_MONITORCheck the value

      - AP_PWR_EN is High andMONI_24V11isLowand when POWER_MONITOR is High

1.1.1.1.1. Set IR_P_ON to High

      - When conditions other than the above apply

        - Set IR_P_ON to Low

  - If Low

1.2.1. Set IR_P_ON to Low.

#### mc_oe_proc

[File]

main/App/km_extend_io.c

[Calling format]

void mc_oe_proc(void)

[Overview]

/MC_OE signal processing

[Arguments]

None

[Return value]

None

[Processing details]

- Do nothing

※As of 2017/5/1, MC_OENo control using

#### mc_pwr_en_proc

[File]

main/App/km_extend_io.c

[Calling format]

void mc_pwr_en_proc(void)

[Overview]

MC_PWR_ENSignal processing

[Arguments]

None

[Return value]

None

[Processing details]

- MONI_24V11Check the value of

  - LowIn case of

    - MC_PWR_ENSet to Low

  - HighIn case of

    - MC_PWR_ENSet to High

#### msw_on_proc

[File]

main/App/km_extend_io.c

[Calling format]

void msw_on_proc(uint8_t state)

[Overview]

MSW_ONSignal processing

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint8_t | state | State of the caller STATE_INT：Call from interrupt STATE_NORMAL：Normal statefromcall of |

[Return value]

None

[Processing details]

- Check state

  - STATE_INT

    - MSW_ONCheck signal

      - High

        - Turn ON power to S800 (s800_power_on())

      - Low

        - Turn OFF the power of S800 (s800_power_off())

  - Otherwise 1.1.

    - MSW_ONCheck signal

      - Low

        - Transition PS-CPU to Sleep mode (BSP_PWR_enter_sleepmode())

      - High

        - Do nothing

#### rx_comp

[File]

main/App/km_i2c.c

[Calling format]

static void rx_comp(uint32_t event)

[Overview]

I2C received data evaluation processing

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint32_t | event | II2C interrupt event |

[Return value]

None

[Processing details]

- Check whether bitARM_I2C_EVENT_TRANSFER_DONEis set in event

  - ARM_I2C_EVENT_TRANSFER_DONEIf the bit is not setReturn without doing anything

- Driver_I2C0.GetStatus()WhendirectionCheck

  - directionis0In case of

    - Set read request flag (i2c_info.read_comp)

    - Set receive size

  - directionis1In case of

    - Set write request flag (i2c_info.write_comp)

    - Set receive size

#### change_state

[File]

main/App/km_i2c.c

[Calling format]

static void inline change_state(int next_state, int* comp, slave_transfer_t transfer, uint8_t* buf, uint16_t size)

[Overview]

Function that switches between I2C receive processing and transmit processing

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | Int | next_state | Specify whether the next transfer is a write request or a read request。 |
| I | int* | comp | Specify which flag to use: read flag or write flag。 |
| I | slave_transfer_t | transfer | Specify the function pointer used for receive or transmit。 ※ The function pointer to specify isDriver_I2C0.SlaveTransmit() orDriver_I2C0.SlaveReceive() only |
| I | uint8_t* | buf | For receive: pointer to store received data. For transmit: pointer to the data buffer to be sent. |
| I | uint16_t | size | Data size to be sent/received |

[Return value]

None

[Processing details]

- Null check for each pointer (comp、transfer、buf)

  - If Null, return without doing anything

- Disable I2C interrupt (HAL_NVIC_DisableIRQ(I2C1_IRQn)is called).

- Store the next requestin the I2C management variable state (i2c_info.state).

- Initialize the flag for read/write requests

- Call the specified function pointer

- Re-enable I2C interrupt (HAL_NVIC_EnableIRQ(I2C1_IRQn)is called).

#### i2c_recv_first

[File]

main/App/km_i2c.c

[Calling format]

void i2c_recv_first(void)

[Overview]

Function called before starting I2C communication

[Arguments]

None

[Return value]

None

[Processing details]

- Set the I2C management variable state to a write request.

* Communication with the S800 always starts with a write request

- Call the I2C receive function.

#### i2c_recv_wait

[File]

main/App/km_i2c.c

[Calling format]

void i2c_recv_wait(void)

[Overview]

Main I2C communication processing

[Arguments]

None

[Return value]

None

[Processing details]

#### checksum_calc

[File]

main/App/km_i2c.c

[Calling format]

uint16_t checksum_calc(void)

[Overview]

Function that calculates and returns the checksum of the main program

[Arguments]

None

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int16_t | Checksum | Return the checksum calculated by the computation (2Byte) |

[Processing details]

- Start address where the main project is placed (0x08000000) to 0x08005000 Add the values in orderAdd

- Reserve the lower 16 bits of the value obtained by addition as the checksum

- Return the checksum(g_checksum_value)

#### hreset_req_proc_handler

[File]

main/App/km_it.c

[Calling format]

static void hreset_req_proc_handler(void)

[Overview]

Interrupt handler for /HRESET_REQ

[Arguments]

None

[Return value]

None

[Processing details]

- hreset_flgSTATE_ACTIVEset to

#### km_exti_callback

[File]

main/App/km_it.c

[Calling format]

static void km_exti_callback(PinType GPIO_Pin ,GPIOEventType event)

[Overview]

ExternalInterrupt handler

* Callback function provided by the HAL driver

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | PinType | GPIO_Pin | GPIO pin number (PA series) |
| I | GPIOEventType | event | Event type |

[Return value]

None

[Processing details]

- Check the event type

  - GPIO_EVENT_EXT_INTRthen go to 2.

  - If not 1.1., end processing

- Check the pin number

  - /HRESET_REQIn the case of a pin

1.1.1./Perform processing for HRESET_REQ (hreset_req_procCall ().

  - AP_PWR_ENIn the case of a pin

1.2.1. AP_PWR_ENPerform processing for (ap_power_en_proc()Call ().

  - POWER_MONITORIn the case of a pin

1.3.1. POWER_MONITORPerform processing for (power_monitor_proc()Call ().

  - Other than the above

#### km_it_init

[File]

main/App/km_it.c

[Calling format]

void km_it_init(void)

[Overview]

KM custom interrupt initialization processing

[Arguments]

None

[Return value]

None

[Processing details]

- Call the GPIO initialization processing.

※During initialization,km_exti_callbackregister ().

#### HAL_TIM_PeriodElapsedCallback

[File]

main/App/km_it.c

[Calling format]

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

[Overview]

Timer interrupt handler

* Callback function provided by the HAL driver

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | TIM_HandleTypeDef | htim | TIMER driver structure |

[Return value]

None

[Processing details]

- NULL check of TIMER driver structure

  - If NULL, end processing

- Check whether it is the TIM3 structure

  - If not TIM3, end processing

- Status registered in the km_timer variable isKM_TIMER_STARTCount down the timer in that state

- When the timer expires, set the status toKM_TIMER_END.

#### km_timer_set

[File]

main/App/km_it.c

[Calling format]

int32_t km_timer_set(KM_TIMER_KIND timer_no,MillsecType timer_val,KM_TIMER_STATE timer_flg)

[Overview]

Timer registration function

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | KM_TIMER_KIND | timer_no | Timer number |
| I | MillsecType | timer_val | Milliseconds (MAX=32768ms) |
| I | KM_TIMER_STATE | timer_flg | Timer flag KM_TIMER_NORMAL KM_TIMER_START KM_TIMER_END |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int32_t | HAL_OK HAL_ERROR | If registration is completed,HAL_OK If registration fails,HAL_ERROR |

[Processing details]

- Timer number error check

  - KM_TIMER_KIND_MAXIf greater than or equal to this, return registration not completed

- Timer value error check

  - KM_TIMER_MAX_TIMEIf greater than or equal to this, return registration not completed

- Timer flag error check

  - KM_TIMER_MAXIf greater than or equal to this, return registration not completed

- km_timerSet status and timer value in the variable

- Return registration completed

#### km_timer_get_state

[File]

main/App/km_it.c

[Calling format]

KM_TIMER_STATE km_timer_get_state(KM_TIMER_KIND timer_no)

[Overview]

Timer status acquisition function

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | KM_TIMER_KIND | timer_no | Timer number |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| KM_TIMER_STATE | KM_TIMER_NORMAL KM_TIMER_START KM_TIMER_END | Timer status |

[Processing details]

- Timer number error check

1.1. KM_TIMER_KIND_MAXIn the above caseKM_TIMER_NORMALis returned

- The specifiedkm_timerstatus is returned

#### adc_moni_5v_start_proc

[File]

main/App/km_adc.c

[Calling format]

void adc_moni_5v_start_proc(void)

[Overview]

5V detection processing

[Arguments]

None

[Return value]

None

[Processing details]

- MONI_5Vstart ADC for

- MONI_5Vacquire ADC value of

- MONI_5Vstop ADC for

- MONI_5VIf the ADC value of  is 4.75 or less, go to 1., if it is 4.75 or more, end the processing

#### adc_moni_5v_off_proc

[File]

main/App/km_adc.c

[Calling format]

void adc_moni_5v_off_proc(void)

[Overview]

5V drop detection processing

[Arguments]

None

[Return value]

None

[Processing details]

- MONI_5Vstart ADC for

- MONI_5Vacquire ADC value of

- MONI_5Vstop ADC for

- MONI_5VIf the ADC value of  is 4.45V or less for 10 consecutive times, perform S800 power OFF control; if it is 4.45 or more, end the processing

*After it has been 4.45V or less for 10 consecutive times, if it returns to 4.75V or more, perform power ON control again

#### BSP_PWR_enter_stopmode

[File]

common/Drivers/BSP/Src/BSP_PWR_STM32F03x_Nucleo.c

[Calling format]

void BSP_PWR_enter_stopmode(void)

[Overview]

Stop mode transition function

[Arguments]

None

[Return value]

None

[Processing details]

- Call the HAL driver’s STOP mode transition function (HAL_PWR_EnterSTOPMode())

#### BSP_PWR_enter_sleepmode

[File]

common/Drivers/BSP/Src/BSP_PWR_STM32F03x_Nucleo.c

[Calling format]

void BSP_PWR_enter_sleepmode(void)

[Overview]

Sleep mode transition function

[Arguments]

None

[Return value]

None

[Processing details]

- Call the HAL driver’s Sleep mode transition function (HAL_PWR_EnterSLEEPMode)

### IAP project

#### main

[File]

iap/App/entry_iap.c

[Calling format]

int32_t main(void)

[Overview]

main function

[Arguments]

None

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int32_t | 0 | A return value is set, but basically it never returns from main |

[Processing details]

- Call the board-specific initialization processing (MX_Init()).

- Change the vector table to SRAM

※Porting of sample code provided by Hakuto

\\10.128.16.110\data1\29P.System Integration SoC Development\03_BSP\07.Outsourcing Management\Hakuto (PS-CPU)\From Hakuto\20160614_Question List, IAP Sample (STM32F030R8)

- Interrupt registration (km_it_init_iap())

- Refresh the watchdog timer (BSP_WDT_Refresh())

- Execute I2C initialization processing (i2c_recv_firstcall () )

- I2C receive processing (i2c_recv_wait())

- Refresh the watchdog timer (BSP_WDT_Refresh())

- Return to 5.

#### rx_comp

Since the function with the same name in the main project has identical processing content,5.9.1.24.rx_compfor reference

#### change_state

Since the function with the same name in the main project has identical processing content,5.9.1.25.change_statefor reference

#### i2c_recv_first

[File]

iap/App/km_i2c_iap.c

[Calling format]

void i2c_recv_first(void)

[Overview]

Function called before starting I2C communication

[Arguments]

None

[Return value]

None

[Processing details]

- Set the I2C management variable state to a write request.

* Communication with the S800 always starts with a write request

- Call the I2C receive function.

- Execute initialization processing of the flash driver (Call Driver_Flash0.Initialize ( NULL ))

- Erase the flash ROM area of the main project

#### i2c_recv_wait

[File]

iap/App/km_i2c_iap.c

[Calling format]

void i2c_recv_wait(void)

[Overview]

Main I2C communication processing

[Arguments]

None

[Return value]

None

[Processing details]

#### flash_write_32

[File]

iap/App/km_i2c_iap.c

[Calling format]

static int32_t flash_write_32(uint32_t addr, void *data )

[Overview]

Write to flash ROM in 32-bit units

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint32_t | addr | Flash address |
| I | void* | data | Pointer to data |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int32_t | ARM_DRIVER_OK ARM_DRIVER_ERROR | Write OK (return value of the driver called internally) Write NG (return value of the driver called internally, or invalid address, or data pointer is NULL) |

[Processing details]

- Address range check

  - MAIN_APPLICATION_ADDRESSLess than and,MAIN_APPLICATION_ADDRESS + MAIN_APPLICATION_SIZEor greater

1.1.1. Return an error

  - In cases other than the above

    - Go to 2.

- Data pointer NULL check

  - If data is NULL

2.1.1. Return an error

  - Other than the above

    - To 3.

- Write to flash (Driver_Flash0.ProgramDataCall )

#### main_project_flash_erase

[File]

iap/App/km_i2c_iap.c

[Calling format]

static void main_project_flash_erase(void)

[Overview]

Main program flash erase function

[Arguments]

None

[Return value]

None

[Processing details]

- MAIN_APPLICATION_ADDRESSfromMAIN_APPLICATION_SIZEErase by sector up to

#### check_sum_calc

[File]

iap/App/km_i2c_iap.c

[Calling format]

static int32_t check_sum_calc(uint32_t data)

[Overview]

Main program data checksum verification function

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | uint32_t | data | MaChecksum data sent from the master side (Value of the program data rewrite completion register) |

[Return value]

| Type | Return value | Contents |
| --- | --- | --- |
| int32_t | ARM_DRIVER_OK ARM_DRIVER_ERROR | Checksum match Checksum mismatch |

[Processing details]

- The data passed as an argumentObtain the checksum value in accordance with the format of the program data rewrite completion register

- Add up the data in the main program area byte by byte (ignore overflow)

- Check the checksum values of 1. and 2.

  - Match

    - ARM_DRIVER_OKis returned

  - Mismatch

    - ARM_DRIVER_ERRORis returned

#### msw_on_proc_iap

[File]

iap/App/km_extend_io_iap.c

[Calling format]

void msw_on_proc_iap(void)

[Overview]

Main switch (primary power) signal processing function

[Arguments]

None

[Return value]

None

[Processing details]

- MSW_ON=0 (Main switch OFF)Check the state

- Program data rewrite completion (0x94)Confirm that the register is set

- MSW_ON=0　Only in this case perform a software reset HAL_NVIC_SystemReset() Execute the function

#### km_exti_callback_iap

[File]

iap/App/km_i2c_iap.c

[Calling format]

void km_exti_callback_iap(PinType GPIO_Pin ,GPIOEventType event)

[Overview]

External interrupt handler for receiving main switch interrupts

[Arguments]

| I/O | Argument | Variable name | Contents |
| --- | --- | --- | --- |
| I | PinType | GPIO_Pin | GPIO pin number |
| I | GPIOEventType | event | Event type |

[Return value]

None

[Processing details]

- Check the interrupt event

- MSW_ON In case of an interrupt, execute the main switch interrupt processingmsw_on_proc_iap()

#### km_it_init_iap

[File]

iap/App/km_i2c_iap.c

[Calling format]

void km_it_init_iap(void)

[Overview]

Interrupt initialization processing

[Arguments]

None

[Return value]

None

[Processing details]

- Interrupt handlerkm_exti_callback_iapRegister ()

# Verification

## ExecutionEnvironment

Required equipment and software

- PC(Keil installation)

- MFP board orPS-CPU evaluation board, virtual environment

- Oscilloscope, etc.

## Verification viewpoints

Below,PS-CPUthe verification viewpoints are shown.

### UT verification execution

- For the newly created codeC0/C1coverage 100%andto perform。

### Startup

- main projectstarts up.

- iap projectstarts up.

### Check various interrupts.

- Interrupts occur and the handler is called.

- The timer interrupt operates correctly with a 1 ms period.

- I2CTransmit/receive interrupts are performed correctly.

### I2CCommunication

- Master deviceCommunication with the master device can be performed correctly according to the format.

- Checksum value can be obtained.

- Version information can be obtained.

### Port control

- Input values of input ports can be correctly referenced.

- Output values output from output ports are correct.

- Ports are correctly controlled according to the input/output conditions.

### IAP

a. Program data transferred from the master can be correctly rewritten.

b. PowerswitchThe rewritten program operates when switching from OFF ⇒ ON.

### Momentary power interruption handling

a. Before turning the power switch ON,Confirm that PowerMonitor is shut down and the power OFF sequence is executed.

b. Power switchONAfterward, confirm that when PowerMonitor is shut down and HRESET_REQ arrives, the power OFF sequence is executed.